## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between core and shell type transformer. |  |
| b) | What is leakage flux? Why a transformer should have low leakage flux? |  |
| c) | What is a conservator? State its function. |  |
| d) | Explain working of transformer on DC supply. |  |
| e) | Explain the advantages and disadvantages of an autotransformer over 2 winding transformer. |  |
| f) | What are the advantages of per unit system? |  |
| g) | List special features of Furnace transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch, explain the following types of transformer windings: - Cross over winding - Helical winding - Continuous disc winding |  |
| b) | Explain the different types of transformer core materials. |  |
| c) | Why cooling is necessary in a transformer? State various methods adopted for cooling. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the phasor diagram of a transformer on No load. |  |
| b) | Draw and explain the equivalent circuit of a transformer referred to primary side. |  |
| c) | A 230/460V transformer has a primary resistance of 0.2Ω, a reactance of 0.5Ω and the corresponding values for the secondary are 0.75Ω and 1.8Ω respectively. Find the secondary terminal voltage when supplying i) 10A at 0.8 p.f. lag ii) 10A at 0.8 p.f. leading. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 50KVA, 440/220V transformer has R1 = 3.45Ω, R2 = 0.009Ω. The value of reactances are X1 = 5.2Ω and X2 = 0.015Ω. Calculate for the transformer: - Equivalent resistance referred to primary side - Equivalent resistance referred to secondary side - Equivalent reactance referred to primary and secondary side - Equivalent impedance referred to primary and secondary side - Total copper loss using 1) Individual resistance of the two windings 2) Equivalent resistance as referred to each side |  |
| b) | Explain different losses taking place in a transformer. |  |
| c) | How does change in frequency affect the operation of a transformer? |  |
|  | -2- | …..2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 20KVA, 440/220V single phase transformer has primary resistance of 0.1Ω and reactance of 0.4Ω. The corresponding secondary values are 0.02Ω and 10.1Ω. Calculate the approximate secondary voltage at full load at 0.8 p.f. lagging. The primary voltage remains the same. |  |
| b) | Find the all day efficiency of 500KVA distribution transformer whose copper loss and iron loss at full load are 4.5KW and 3.5KW respectively. During a day of 24 hours it is loaded as under: No of hrs.; Loading in KW; p.f.. 6; 400; 0.8. 10; 300; 0.75. 4; 100; 0.8. 4; 0; -. |  |
| c) | Explain the conditions that must be fulfilled for the successful parallel operation of 1 phase and 3 phase transformers. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two single phase transformers with equal turns have impedance of (0.5 +j3) Ω and (0.6 + j10) Ω with respect to the secondary. If they operate in parallel, determine how they will share total load of 100KW at 0.8 p.f. lagging? |  |
| b) | Explain in detail the construction and working of an autotransformer. |  |
| c) | Write a short note on Scott connected transformer. |  |

***********