## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Architectural Engg.

Subject: Estimation & Costing(4529)/(AE503) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | State the basic differences between estimation & billing. | (3) |
| b) | Draw a format and show contents of measurement sheet and abstract sheet. | (4) |
| c) | Define the term ‘Rate analysis’ and state its purpose. | (4) |
| d) | State how inaccurate measurements taken at site affect the budget of house owner. | (4) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the specification for following item of works. Make suitable assumptions. i) Excavation for foundation ii) Plain cement concrete in foundation |  |
| b) | State and explain any two features of item wise estimate. |  |
| c) | State the units of measurements for following items of work: i) Formwork for R.C.C. ii) Reinforcement in R.C.C. iii) Damp proof course iv) R.C.C. Chajja v) Stone work in Arch vi) R.C.C. foundation vii) Half brick partition wall viii) water proofing on terrace ix) Tiled roofing x) Wood work in door & window frames xi) Painting of M.S. grills xii) Fixing of sanitary fixtures |  |
| Q.No.3. | Calculate quantities of any two of the following items of works and prepare a bill of quantities. Make suitable assumptions. Refer Fig.1. | (12) |
|  | i) Laterite masonry in superstructure in cement mortar 1:5 |  |
|  | ii) External cement plastering 18mm thick in 2 coats |  |
|  | iii) Plain cement concrete in plinth |  |
|  | Only for students of Revised scheme: |  |
| Q.No.4. | Answer the following Questions: | 3 x 4 = 12 |
| a) | Define the term ‘estimation’ with respect to construction. Mention any three qualities one should possess to be able to estimate the cost accurately. -2- | …….2/- |
| b) | State the importance of schedule of rates. Mention any two salient features of schedule of rates and any two factors that can fluctuate the rates of item of work. |  |
| c) | Mr.Sohan is given a job of maintaining measurement book at construction site. State any four rules he should follow while writing measurement book. |  |
| Q.No.5. | Calculate quantities and prepare a measurement sheet for any two of the following: | (12) |
|  | i) Proposed compound wall (Ref. fig.2) |  |
|  | ii) R.C.C. water tank (Ref. Fig.3) |  |
|  | iii) Internal plumbing work for Indian W.C. in an area of 1m x 1.3m. Exclude plastering & floor finishing work. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Calculate quantity of excavation for foundation by centre line method. (Ref. Fig.1) | (4) |
|  | ### OR |  |
| b) | Calculate quantity of excavation for foundation by long wall short wall method. (Ref. Fig.1) | (4) |
| c) | Mention the four components considered for rate analysis of any item of works. | (4) |
| d) | Calculate quantities of materials (i.e. cement, sand, aggregate) for R.C.C. water tank shown in fig.3. | (4) |
|  | ### OR |  |
| e) | Prepare rate analysis for following works: i)12mm thick Cement plastering 1:5 on new brickwork (100 sq.m.) ii) Distempering two coats on a primer to new plaster (100 sq.m. area) Use following data: Coverage Material cost Primer; 20 sq.m./litre; Rs.100/litre. Distemper; 8 sq.m./Kg; Rs.820/20kg. Cement bag; -; Rs.375/bag. Sand; -; Rs.1500/m3. | (4) |
|  | Only for students of Old scheme: |  |
| Q.No.4. | Answer the following Questions: | 4 x 3 = 12 |
| a) | State different methods of approximate estimates. Write salient features of each. Which of these is more useful in absence of detail plan. |  |
| b) | State the importance of knowledge of estimation & costing to an architect. |  |
| c) | State the difference between Revised estimate and Supplementary estimate. |  |
| d) | State the requirements of a detailed estimate. |  |
|  |  | ……3/- |
|  | -3- |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Define i) Contingencies ii) Work charged establishment with example of each. State the amount reserved in estimate towards contingencies and work charged establishment. | (6) |
|  | ### OR |  |
| b) | State the six components that constitute a complete estimate. | (6) |
| c) | Prepare bill of quantities for any one of the following works: - Proposed compound wall shown in fig.2 - R.C.C. water tank shown in fig.3 | (6) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the four components considered for analysis of rates. |  |
| b) | Calculate quantity of materials (sand, cement, aggregate) for R.C.C. water tank shown in fig.3. |  |
| c) | Prepare rate analysis for following works: i) 12mm thick cement plastering 1:5 on new brickwork (100 sq.m.) ii) Distempering two coats on a primer to new plaster (100 sq.m. area) Use following data: coverage material cost Primer; 20 sq.m./litre; Rs.100/litre. Distemper; 8 sq.m./Kg; Rs.820/20Kg. Cement bag; -; Rs.375/bag. Sand; -; Rs.1500/m3. |  |