## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Draw the projections of a line 70 mm long inclined at 30º to HP and its top view appears to be inclined at 60º to VP. One of the ends of the line is 45mm above HP and 60 mm infront of VP. Draw its projections by change of ground line method. |  |
| b) | A hexagonal lamina of 30mm rests on HP such that its surface is inclined at 45º to HP and one of its diagonals is inclined at 45º to VP. Draw its projections using auxiliary plane method. |  |
| c) | Draw the top and front views of a right circular cylinder of base 45mm diameter and 60mm long when it lies on HP such that its axis is inclined at 30º to HP and the axis appears to be perpendicular to VP in the top view. Draw the projections using auxiliary plane method. |  |
| Q.No.2. | Answer any one of the following Questions: |  |
| a) | A square pyramid base 40mm side and axis 65mm long has its base on HP and all the edges of the base are equally inclined to VP. It is cut to the truncated shape with the truncated surface inclined at 45º to its axis bisecting it. Draw the development of the Pyramid. | (10) |
| b) | A square Pyramid, side of base 50mm and height of axis 75mm stands on its base in the HP, with an edge of base perpendicular to VP. A horizontal circular hole of diameter 24mm is drilled through the triangular faces of the pyramid and the axis of the hole and Pyramid intersect at 30mm above the base. Draw the development of the lateral surface of the pyramid with hole. | (10) |
| Q.No.3. | Answer any one of the following Questions: |  |
| a) | A vertical square prism, base 60mm side is completely penetrated by a horizontal square prism, base 40mm side, so that their axes intersect. The axis of the horizontal prism is parallel to VP, while the faces of the two prisms are equally inclined to VP. Draw the projections of the solids showing lines of intersection. Assume suitable lengths of the prisms. | (12) |
| b) | A vertical cylinder of 80mm diameter and 100mm long is penetrated by a horizontal cylinder of 60mm diameter and 100mm long. The axes of both the cylinders are 10mm apart and parallel to VP. The axis of the horizontal cylinder lies infront of the axis of the vertical cylinder and passes at the mid height of the vertical cylinder. Draw the intersection curve. | (12) |
| Q.No.4. | Fig.1 shows the orthographic views of a component in first angle method of projection. Draw suing the same angle of projection: i) Sectional front view ii) Sectional side view iii) Top view. | (16) |
|  | -2- | ……2/- |
| Q.No.5. | Answer the following Questions: |  |
| a) | Give the conventional representation for the following: i) Concrete ii) Insulating material iii) Wood iv) Liquid | (8) |
|  | OR |  |
| a) | With the help of suitable free hand sketch show the following: i) Half section ii) Off-set section iii) Revolved section iv) Partial section | (8) |
| b) | Draw by double line method a 20mm diameter pipe showing: i) Lateral ii) 45º Elbow iii) Union | (9) |
| Q.No.6. | Answer any one of the following Questions: | (8) |
| a) | The object shown in Fig.2 is to be fabricated by welding pieces A, B, and C in the proper relative positions. Draw the orthographic views of the object and indicate the [roper welding symbols. |  |
| b) | Show the welding symbols for the following types of welds: - Square butt weld v) Slot weld - Spot weld vi) Backing weld - Single bevel butt weld vii) Fillet weld - Single J butt weld viii) Seam weld |  |