## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE/MED/COMP

Subject: Basic Electronics(4132) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is intrinsic and extrinsic semiconductor? Draw necessary diagrams. |  |
| b) | Define ,  for a transistor and give relations between them. |  |
| c) | Explain thermionic and secondary emission. |  |
| d) | Define transconductance and amplification factor for JFET. Write their equations. |  |
| e) | Explain common collector configuration and give its application. |  |
| f) | The emmitter current IE in a transistor is 2mA. If the leakage current is 5A and dc=0.985, calculate IC and IB. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of P-N Junction diode under forward biased condition. Draw circuit diagram and necessary graph. |  |
| b) | Explain with neat circuit diagram and waveforms, the working of bridge rectifier with capacitor filter. |  |
| c) | Explain the working of  filter with necessary circuit diagram and waveforms. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat diagram crystalline structure of P type semiconductor. Also draw energy band diagram. |  |
| b) | Explain Zener breakdown and avalanche breakdown. |  |
| c) | In a centre-tapped rectifier, diodes are assumed to be ideal, transformer primary to secondary ratio is 5:1, 230V A.C. mains is given to primary and load resistance of 100 is connected. Draw the circuit diagram and calculate i) d.c. o/p voltage ii) PIV iii) Rectification efficiency. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of NPN transistor with necessary diagrams. |  |
| b) | Draw and explain I/P and O/P characteristics of common basic configuration. Draw necessary circuit diagram. |  |
| c) | Explain the need for biasing of a transistor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) If dc =0.99, calculate dc. ii) Collector leakage current in CE configuration is 300A. If now the transistor is connected in CB configuration, what will be the leakage current if =120. iii) For a transistor IB =20A, IC=2mA, =80, calculate ICBO. |  |
| b) | Explain the terms Zi, Zo, Av, Ai for a transistor. |  |
| c) | Draw fixed bias circuit and explain it. Why is this circuit unsatisfactory for transistor biasing? |  |
|  | -2- | ……2/- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail important JEET parameters with the help of characteristics. |  |
| b) | Explain construction, working and characteristics of vacuum triode tube. |  |
| c) | Compare between: - BJT and JFET - MOSFET and JFET |  |

***********