## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction to Thermodynamics(4334)/(SH404) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by property of a system? Distinguish between extensive and intensive properties of a system. |  |
| b) | What do you understand by a “Thermodynamic Process”? What is reversible process? |  |
| c) | Explain the following terms related to Internal Combustion engines: i) Stroke length ii) Clearance volume |  |
| d) | Explain the term Dryness fraction of steam. |  |
| e) | List a few important factors, which govern the transfer of heat by conduction. |  |
| f) | Define: i) “Coefficient of Performance” of a Refrigerator ii) Ton of Refrigeration |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain First law and Second law of Thermodynamics. |  |
| b) | A gas occupies a volume of 0.1 m3 at a temperature of 20C and a pressure of 1.5 bar. Find the final temperature of the gas if it is compressed to a pressure of 7.5 bar and occupies a volume of 0.04 m3. |  |
| c) | Draw the Diesel cycle on P-V diagram and explain the various processes. Write an expression to determine the Heat supplied to the air. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain constant volume heating process. Represent the process on P-V diagram and T-S diagram. Write an expression for change in Internal Energy and Heat supplied. |  |
| b) | A quantity of air has a volume of 0.4m3 at a pressure of 5 bar and temperature of 80C. It is expanded in a cylinder at a constant temperature to a pressure of 1 bar. Determine the amount of work done by the air during expansion. |  |
| c) | A volume of 0.5 m3 of gas at a pressure of 10 bar and 200C is expanded in a cylinder to 1.2m3 at a constant pressure. Calculate the amount of work done by the gas and the change in Internal energy. Assume Cp = 1.005 KJ/KgK and Cv = 0.712 KJ/KgK. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the working of Two-stroke diesel engine. |  |
| b) | List the various systems of Internal Combustion Engine. Explain any one system. -2- | …….2/- |
| c) | The following data refers to a test on a two-stroke cycle oil engine: No. of cylinders : 1 Engine speed : 480 rpm Cylinder diameter : 0.3m Stroke length : 0.4m Indicated mean effective pressure: 650 KPa Brake Torque : 750 Nm Oil consumption : 9.2 Kg/hr Calorific Value of fuel : 45000 KJ/kg Calculate i) Indicated Horse power in KW ii) Brake Horse power in KW iii) Indicated Thermal Efficiency |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Determine the condition of steam in each of the following cases: i) At a pressure of 15 bar and 220C ii) At a pressure of 6 bar and enthalpy of 2600 KJ/Kg iii) At a pressure of 12 bar and specific volume 0.16321 m3/Kg |  |
| b) | Determine the quantity of heat required to produce 5 Kg of steam at a pressure of 6 bar and a temperature of 25C, when the steam is wet having a dryness fraction of 0.85. |  |
| c) | A hot plate 1m x 1.5m is maintained at 300C. Air at 20C blows over the plate. If the convective heat transfer coefficient is 20 W/m2 C, calculate the rate of heat transfer. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the simple vapour compression refrigeration system. |  |
| b) | In a vapour compression refrigeration system, the refrigerant enters the compressor in dry saturated state and leaves the condenser in sub cooled state. Represent the above cycle on P-h and T-S diagram. |  |
| c) | A refrigeration machine using R12 as refrigerant operates between -7C and 36C, the vapour is dry at the entry of compressor and there is no under cooling in the condenser. The vapour leaves as refrigerant gas at 46C from compressor. Determine theoretical COP of the cycle using following properties. Sat. Temp.C; Pressure (bar); Enthalpy Liquid KJ/Kg; Enthalpy Vapour KJ/Kg; Entropy of Vapour KJ/KgK. 36; 9; 456.4; 585.3; 4.74. -7; 2.5; 412.4; 570.3; 4.76. |  |
|  | Take Cp of vapour as 0.67 KJ/KgK |  |

***********