## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Structure Design(SH721)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write a short note on panting. |  |
| b) | Explain the theorem of three moment. |  |
| c) | What is fixed beam? List down some of its advantages. |  |
| d) | What are assumptions made in Euler’s column theory? |  |
| e) | What are the advantages of FEM? |  |
| f) | Explain with a neat sketch Hogging and Sagging. |  |
| g) | Write a short note on node location system. |  |
| h) | Write a short note on Transverse stress associated with ship. |  |
| Q.No.2. | A steel compound column 4m long is build up by two steel joints of ‘C’ section joined by two steel plates 300mm x 10mm as shown in fig.(a). Find the Rankine crippling load if both ends of the column are hinged. Pc = 35MPa, Rankine constant = 1/7500 | (12) |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | A beam of uniform section and 6m span built in at the ends. An uniformly distributed load of 3KN/m runs over the left half of the span and there is in additional concentrated load of 4KN at 1.5m from right support as shown in fig.(b). Determine the fixing moment at the ends and reactions. |  |
| b) | A fixed beam AB of span 6m is carrying a UDL of 5KN/m over the left half of the span. Find the fixing moment and support reactions. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | A continuous beam ABCD, simply supported at A, B, C & D is loaded as shown in fig.(C). Find the moment over the Beam and draw bending moment. |  |
| b) | Evaluate the bending moment and support reaction of a beam loaded as shown in fig.(d). Also draw bending moment diagram. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | For a given mid ship section shown in fig.(e), Vertical bending moment = 300 MN-m, Horizontal bending moment = 150 MN-m, Angle of Heel = 0. Calculate stress at different location (1-9) shown in the figure. E = 206.9 x 109Pa, L = 130m, B = 20m, D = 13m |  |
| b) | For a given mid ship section shown in fig.(e), Vertical bending moment = 300 MN-m, Horizontal bending moment = 150 MN-m, Angle of Heel = 15. Calculate stress at different location (1-9) shown in fig.(e). E = 206.9 x 109Pa, L = 130m, B = 20m, D = 13m |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by FEM? Briefly explain the steps involved in FEM. -2- | ……2/- |
| b) | Briefly explain node numbering scheme. |  |
| c) | M/I values are given for a vessel as follows (MN-m/m4) Station; 0; 1; 2; 3; 4; 5; 6; 7; 8. M/I; 0; 0.8; 1.9; 2.7; 3.8; 2.6; 1.5; 0.4; 0. Calculate the deflection at the keel amidship L = 160m, E = 200GN/m2. |  |