## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Production Processes(4059) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How are moulding sands classified? How do they differ from each other? |  |
| b) | Name the three basic methods of centrifugal casting. |  |
| c) | What is the phenomenon of ‘Grain Flow’ in forging? |  |
| d) | What is the minimum number of teeth which a gear can have? Which of the following standard involute cutter has the greater number of teeth, Cutter No.2 or Cutter No.6. |  |
| e) | Name the three different types of flames which can be obtained by the combustion of a mixture of oxygen and Acetylene. |  |
| f) | What is meant by ‘throwing power’ in electrolysis? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How does a ‘Skin dried mould’ differ from a ‘Dry sand mould’? |  |
| b) | What is the role of a ‘Riser’ in the context of a sand casting mould? |  |
| c) | What are the precautions to be taken while pouring molten metal into a sand mould in a large foundry? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using a neat sketch explain the Hot chamber die casting process. |  |
| b) | Draw neat sketches of any three types of marking tools used in sheet metal work. |  |
| c) | How does the operation of ‘Punching’ differ from ‘Blanking’ as a sheet metal working process? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is ‘Closed die forging’? What are the precautions to be taken during the process of Closed die forging? |  |
| b) | Using a neat sketch explain the ‘Board Hammer drop forging’ process. |  |
| c) | What is the difference between the ‘Manual stroking’ and ‘Power stroking’ method of honing? What should be the factors that would decide on the choice between the two? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the method of ‘Gear shaping’ as a cutter gear generating process. |  |
| b) | What are the three types of ‘Hobbing’? Show diagrams highlighting the difference between them. |  |
| c) | Write a note on ‘Gear shaving’ as a Gear finishing method. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How does the ‘High pressure system’ of the oxyacetylene gas welding process differ from the ‘Low pressure system’? -2- | ……2/- |
| b) | What are the different variations of the Arc welding process? Explain any one of them in brief. |  |
| c) | What is ‘Soldering’? Where is it used? What is the equipment required for soldering? |  |

***********