## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(4335)/(SH401) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Base and grain capacity |  |
| b) | Side Launching |  |
| c) | Capacity Plan |  |
| d) | Wall sided formula |  |
| e) | Load Curve and buoyancy curve |  |
| f) | Margin line |  |
| Q.No.2. | Describe with the sketch launching curves used in stern launching of vessel. What are the important features of the curves? | (8+4) |
| Q.No.3. | A uniform shaped barge 40m x 12m beam is divided into four cargo compartments each 10m long. The barge is loaded with 600 tonnes of iron ore level stowed as follows: No. 01 hold – 135 tonnes; No. 02 hold – 165 tonnes; No. 03 hold – 165 tonnes; No. 04 hold – 135 tonnes. The loaded barge floats in fresh water on an even keel draught of 1.75m. Construct the curve of shearing force and bending moment for this condition and also find the position and value of the maximum bending moment for the still water condition and state its type. | (12) |
| Q.No.4. | A box shaped vessel 100m long x 20m wide x 12m deep is floating in salt water on an even keel at 6m draft. A forward compartment 8m long, 20m wide and containing cargo of permeability 25 percent is bilged. Find the new drafts after flooding. | (12) |
| Q.No.5. | A vessel is loaded up ready for departure. KM is 11.9m and KG is 9.52m with a displacement of 20,550 tonnes. From the ship’s cross curves of stability, the GZ ordinates for a displacement of 20550 tonnes and a VCG of 8m above base are as follows. Angle of heel (); 0; 15; 30; 45; 60; 75; 90. GZ ordinate; 0; 1.10; 2.22; 2.60; 2.21; 1.25; 0.36. Using the information, construct the ships for this condition of loading and determine the following: - Maximum righting lever GZ - Angles of heel at which this maximum GZ occurs 3. Range of stability | (12) |
| Q.No.6. | Answer the following Questions: |  |
| a) | A box shaped vessel 65m x 10m x 6m is floating upright on an even keel at 4m draught in salt water. GM = 0.6m. Calculate the dynamic stability at 20% heel. | (7) |
| b) | With a neat sketch derive the equation to find the moment of statical stability using Atwoods formula. | (5) ……2/- |
|  | -2- |  |
| Q.No.7. | Answer the following Questions: |  |
| a) | With a neat sketch explain the procedure to find the stress in deck and bottom of the vessel when bending moment is known. | (10) |
| b) | How section modulus of the vessel can be increased by keeping the weight of the vessel to minimum? | (2) |

***********