## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare processors 80386 and 80486 on the basis of number of address & data lines and speed. |  |
| b) | What is the need of memory segmentation in 8086? |  |
| c) | What is an interrupt? Name different interrupts of 8051. |  |
| d) | Write the instruction to save data 87H to register R4 of Bank 2 of 8051. |  |
| e) | What is the difference between microprocessor and microcontroller? |  |
| f) | What are SFRs? Where are they located? Name any two SFRs of 8051. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain flag register of 8086. |  |
| b) | Draw and explain bus interface unit of 8086. |  |
| c) | Draw block diagram of 8051 and explain in brief. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain various bits of TCON register of 8051. |  |
| b) | What is bit addressable RAM of 8051? Write the address range of bit addressable RAM and explain use of it with example. |  |
| c) | Describe steps to transfer data serially in 8051. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are addressing modes? Explain addressing modes of 8051 with examples. |  |
| b) | How many parallel input/output ports are available in 8051? Draw block diagram of any one of them and explain read and write operations. |  |
| c) | Draw the structure of internal RAM of 8051, along with the address range and explain in brief. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain PUSH and POP instructions of 8051. |  |
| b) | What is subroutine? Explain CALL and RETURN instructions of 8051. |  |
| c) | Write an assembly language programme for 8051 to copy 10 data bytes stored in internal RAM starting from 20H onwards to the other location of internal RAM starting from 40H onwards in reverse order. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain following instructions: - CJNE A, #n, radd - JNB address |  |
| b) | Explain unconditional jump instructions of 8051. |  |
| c) | Write the instructions for following for 8051: - Complement 3rd bit of port 1 - Exchange data between register RO and R1 |  |
| d) | Explain with example, DA A of 8051. |  |

***********