## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Design & Drawing of Automobile Components(4067) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the general considerations and procedure in Design? |  |
| b) | Explain any four types of keys used in couplings with neat sketches |  |
| c) | What is “Nipping” in leaf spring? Discuss its role in spring design. |  |
| d) | Give the functions of couplings and clutches? How does a coupling differ from a clutch? |  |
| e) | Why is a hollow piston preferred to a solid one? |  |
| f) | Explain working stress. Sketch a stress strain diagram for mild steel & explain. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A link of a machine is subjected to a force of 20KN as shown in fig.1. The link is made of Grey cast iron having allowable strength of 100N/mm2. Determine the dimension of the cross section. |  |
| b) | A mild steel shaft as shown in fig.2 receives 25KW power at 500 rpm through a belt pulley of diameter 500mm. Ratio of tension of the belt is 3:1 and weight of the pulley is 50Kg. Determine the diameter of the shaft if permissible shear stress for the material is 40MPa. |  |
| c) | An overhang crank, as shown in fig.3 carries a tangential load of 10KN at the centre of the crankpin. Find the maximum principal stress and the maximum shear stress at the centre of the crankshaft bearing. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel plate subjected to a force of 5KN and fixed to a channel by mean of three identical bolts as shown in fig.4. The bolts are made from plain carbon steel 45C8 (yt=380 N/mm2) and the factor of safety is 3. Determine the size of the bolts. -2- | …….2/- |
| b) | A rigid flange coupling is used to transmit 15KW power at 720 rpm between two shafts. The shafts, keys and bolts are made of plain carbon steel 30C8 (yt =400 N/mm2) and the factor of safety is 3. The yield strength in compression can be taken as 150% of the tensile yield strength. The flanges are made of grey cast iron FG200 (ut =200 N/mm2) and the factor of safety is 6. The keys have square cross section. Design the coupling. |  |
| c) | The standard cross section for a flat key, which is fitted on a 50mm diameter shaft is 16mm x 10mm. The key is transmitting 475 N-m torque from the shaft to the hub. The key is made of commercial steel (yt = yc = 230N/mm2). Determine the length of the key, if the factor of safety is 3. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a close coiled helical compression spring for a service load ranging from 2250N to 2750N. The axial deflection of the spring for the load range is 6mm. Assume a spring index of 5. The permissible shear stress intensity is 420MPa and modulus of rigidity G=84KN/mm2. Neglect the effect of stress concentration. |  |
| b) | The laminated spring in a truck has 12 leaves, two of which are full length. The spring supports are 1.05m apart and the central band is 85mm wide. The central load is 54KN and the permissible stress of the material is 280N/mm2. Design the spring if the ratio of total depth to the width of the spring is 3. Also determine the deflection of the spring. |  |
| c) | A rigid coupling is used to transmit 25KW at 720 rpm. There are four bolts and P.C.D. of the bolts is 125mm. The bolts are made of steel 45C8 (yt =380 N/mm2) and factor of safety is 3. Determine the diameter of the bolts. Take ys=0.577yt. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) A single plate clutch with both sides effective is engaged by means of 6 helical springs. The coefficient of friction is 0.3 and the mean diameter of the friction surface is 230mm. Find the axial force exerted by each spring if the torque transmitted by the clutch is 245 N.m. ii) List the different types of clutches. Also discuss the clutch lining materials. |  |
| b) | The connecting rod of a four stroke diesel engine is made of M.S. It is of circular section and of length 550mm. The diameter and stroke of cylinder are 150mm and 240mm respectively. The maximum combustion pressure is 4.7N/mm2. Determine the diameter of the rod to be used for a factor of safety of 3 with a material having yield point of 330N/mm2. -3- | ……3/- |
| c) | Determine the diameters of the hollow push rod made of mild steel of an I.C. engine if the maximum force exerted by the push rod is 1500N. The length of the push rod is 50cms. Take F.O.S=2.5 and the end fixity coefficient as 2. Take E=210KN/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a crown thickness of a C.I. piston for a single acting 4 stroke engine for the following specification: Cylinder bore = 102mm, stroke = 120mm, maximum gas pressure = 4.8MPa, B.m.e.p = 0.6Mpa, speed = 2200 rpm, fuel consumption = 225 gm/KW-hr, Allowable tensile stress for C.I. = 37MPa, Temperature difference between centre and edge of crown = 220°C, Heat conductivity factor = 47W/m°C, H.C.V. of fuel 41880 KJ/Kg. |  |
| b) | A multi disc clutch employs 3 steel and 2 bronze discs having outer diameter 30cm and inner diameter 20cms. If the coefficient of friction is assumed as 0.22, find the axial pressure and horsepower transmitted if the unit normal pressure is 0.13N/mm2 and rpm = 750. |  |
| c) | i) Explain allowances, tolerances and fits. ii) Name different types of fits and explain their significance. |  |