## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./SB

Subject: Fiber Reinforced Polymers(IE651)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by a Matrix and a Reinforcement? What is their basic function? |  |
| b) | What are Kevlar fibers? |  |
| c) | Why are promoters added to polymer matrix? |  |
| d) | Describe contact molding process. |  |
| e) | Describe adhesive bonding method of joining two materials. |  |
| f) | What is resin peak exotherm temperature? |  |
| g) | List the tools and materials required for manufacturing of FRP components. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various constituent phases of composites with a neat sketch. |  |
| b) | Why is surface treatment required for fibers? What are glass fibers? |  |
| c) | What are the characteristics & applications of various thermosetting matrix systems? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are Prepegs? How are they manufactured? |  |
| b) | Explain Filament winding process with a neat sketch. |  |
| c) | Explain the machining method of post processing the composites. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the hydrostatic test used for inspection of FRP components. |  |
| b) | Explain the factors influencing design of FRP product. |  |
| c) | How can the waste developed during manufacture of FRP be disposed? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the rotational molding process of manufacturing composites with a neat sketch. |  |
| b) | Write a note on selection of raw materials while designing a FRP product. |  |
| c) | State various characteristics of fiber reinforced composites. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Carbon Fibers |  |
| b) | Fillers |  |
| c) | Safety precautions during FRP manufacture |  |
| d) | Bag molding |  |
| e) | Pipe burst test |  |

***********