## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)/(EE502) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three points of distinction between DIAC and TRIAC. |  |
| b) | Explain in brief dv/dt triggering of SCR. |  |
| c) | Draw the circuit diagram & relevant waveforms for a single phase full wave converter with resistive load. (M2 connection) |  |
| d) | What is the difference between series Inverter and modified series Inverter? |  |
| e) | Explain in brief the operation of class A chopper. |  |
| f) | Write a short note on solid state switches. |  |
| g) | Write a short note on stator voltage control for speed control of single phase Induction motor. |  |
| h) | Draw the circuit diagram of a Snubber circuit for an SCR. How does it provide dv/dt protection? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the symbol and characteristics of i) IGBT ii) GTO iii) MCT. |  |
| b) | Write a short note on General purpose diodes. |  |
| c) | Write a short note on Schottky diodes. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw & explain two transistor model of a thyristor & state the expression of anode current flowing during its conduction clearly indicating each term used in expression. |  |
| b) | What do you understand by commutation of a thyristor? State the various methods of commutation of a thyristor and explain any one in detail. |  |
| c) | Draw static & dynamic equalizing circuits for thyristor in series and explain their operation. |  |
| d) | Explain any one mode of operation of chopper drive of your choice with the help of neat circuit diagram & waveforms. students of old scheme) | (only for |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram & waveforms explain the working principles of single phase half wave converter with RL load of freewheeling diode. |  |
| b) | Draw the circuit diagram & waveforms of three phase half wave converter for inductive load. Also write the expressions for average and rms values of voltage. |  |
| c) | Write a short note on operating principle of Cycloconverter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram & waveforms explain ON-OFF control method used in AC voltage controllers. |  |
| b) | With the help of neat diagram & waveforms explain the operation of class B chopper. -2- | ……..2/- |
| c) | With the help of neat diagram, explain the working principle of Parallel Inverter. |  |
| d) | Differentiate between regenerative braking & rheostatic braking in dc drives. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the working of Boost regulator. |  |
| b) | Write a short note on frequency control method used for ac motor control. |  |
| c) | Write short notes on i) di/dt protection ii) Cooling & heat sinks for power electronic devices. |  |

***********