## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Engineering & Technology

Subject: Applied Physics-II(GN202) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define 1 joule. |  |
| b) | State Ohm’s law. |  |
| c) | Define electric power. |  |
| d) | What is Self induced emf? |  |
| e) | Draw a symbol of NOT gate. |  |
| f) | Convert the binary number 1101 into decimal number. |  |
| g) | What are ultrasonic waves? |  |
| h) | Define Centripetal force. |  |
| i) | Why curved roads are banked? |  |
| j) | Define velocity ratio. (only for Mechanical Engg. & Allied branches) |  |
| j) | What is the function of transformer in rectifier circuit? Electronics Engg. & Allied branches) | (only for |
| Q.No.2. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain direct and remote action forces. Give an example in each case. |  |
| b) | State law of conservation of energy. Find the potential energy by a body of 600N at a height of 4m. |  |
| c) | A force acting on a body gradually increases from 0 to 50N and displaces it through 2m. Find the work done graphically. |  |
| d) | Define (i) Work (ii) Power. |  |
| e) | How many litres of water can be lifted in 10 minutes to a height of 20m by a pump of 75KW? Efficiency of pump is 80%. |  |
| f) | Obtain an expression for effective resistance when two resistances R1 and R2 are connected in series. |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | A wire of length 2.4m, radius 0.42mm has a resistance of 15 ohm. Calculate specific resistance of the wire. |  |
| b) | Explain effect of temperature on resistance. Define temperature coefficient of resistance. |  |
| c) | Three resistance 7Ω, 12Ω & 21Ω are connected in i) series ii) parallel. Find the effective resistance in each case. |  |
| d) | A battery of emf 12V and of internal resistance 1Ω is connected to a resistance of 5Ω. Calculate current in the circuit. |  |
| e) | Draw a circuit diagram and explain comparison of EMF of 2 cells by single cell method using potentiometer. |  |
| f) | Calculate energy consumed in KWh for 10 days when 2 electric lamps of 60W each are lighted for 5 hours a day and an electric iron of 100W for 2 hours a day. |  |
|  | -2- | …..2/- |
| Q.No.4. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | i) State Lenz’s law. ii) Define one henry. |  |
| b) | Explain Faraday’s experiment of electromagnetic induction. State the conclusion. |  |
| c) | The magnetic flux associated with a coil of 50 turns changes from 42 x 10-5 wb to 2 x 10-5 wb in 0.01s. Calculate the induced emf. |  |
| d) | Convert following decimal numbers into binary number: i) 23 ii) 15 |  |
| e) | Draw AND gate and write boolean expression. Write its truth table. |  |
| f) | Define resonance. Explain with one example. |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain formation of beats with a neat diagram. |  |
| b) | State any three engineering applications of ultrasonic waves. |  |
| c) | A hall of volume 2500m3 has a reverberation time of 2 seconds. If the area of the absorbing surface is 1660m2, determine the coefficient of absorption. |  |
| d) | Obtain an expression for angle of banking. |  |
| e) | A 2.4 Kg mass is tied to a string at one end and rotated in a horizontal circle of radius 0.8m about the other end. If the breaking tension in the string is 250N, find the maximum speed at which the mass can be rotated. |  |
| f) | Two bodies of masses 20 Kg and 40 Kg are placed 20cm apart. Find the force of attraction between them. |  |
|  | only for Mechanical Engg. and allied branches: |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Define acceleration due to gravity. Show that g=GMR2. |  |
| b) | State the advantages of single fixed pulley and single movable pulley. Define Mechanical Advantage. |  |
| c) | Define i) Ideal machine ii) Efficiency of machine. |  |
| d) | Draw and explain second order pulley system. Write an expression for Velocity Ratio. |  |
| e) | In a certain weight lifting machine weight of 1000N is lifted by an effort of 25N. While the weight moves up by 100mm, the point of application of effort moves by 8m. Find the Mechanical Advantage, Velocity Ratio and Efficiency of machine. |  |
| f) | What load will be lifted by an effort of 12N, if the velocity ratio is 18 and efficiency of the machine at this load is 60%? |  |
|  | only for Electronics Engg. and allied branches: |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Define acceleration due to gravity. Show that g=GMR2. |  |
| b) | Define i) Knee voltage ii) Break down voltage. |  |
| c) | Draw diode in forward bias and explain. Draw VI characteristic in forward bias. |  |
| d) | Explain half wave rectifier with diagram. |  |
| e) | Draw and explain full wave bridge rectifier. |  |
| f) | If no load voltage is 25V and full load voltage is 18V, what is the percentage regulation? |  |

***********