## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-I(4022)/(FE601) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the concept of use of influence lines. |  |
| b) | Explain different cases of structures subjected to bending and axial loading with sketches. |  |
| c) | Show the limit of eccentricity for rectangular section subjected to eccentric loading to avoid tension at base. |  |
| d) | Explain the concept of pure torsion. |  |
| e) | What are Principal planes and Principal stresses? |  |
| f) | Explain statistically indeterminate structures with example. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pier is loaded with a vertical load of 30KN at 2.5m from Y-Y axis and 1.2m from X-X axis in the fourth quadrant of the pier of section 8m x 6m. Find the stresses at each corner. |  |
| b) | The line of thrust in a compression testing specimen 14.32mm diameter is parallel to the axis of the specimen but is displaced from it. Calculate the distance of the line of thrust from the axis when the maximum stress is 15% greater than the mean stress on a normal section. |  |
| c) | A short column of external diameter 150mm and internal diameter 100mm carries an eccentric load. Find the greatest eccentricity, which the load can have without producing tension on the cross-section of the column. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A point in a strained material is subjected to a tensile stress 60N/mm2, a compressive stress of 40N/mm2 acting on two mutually perpendicular planes and a shear stress of 10N/mm2 acting on these planes. Find the normal stress, tangential stress and resultant stress on a plane inclined to 30 with the plane of the compressive stress. |  |
| b) | At point in a material, the principal stresses are 100N/mm2 and 50N/mm2 both tensile. Find the resultant stress on a plane inclined at 60 to the axis of major principal stress. |  |
| c) | Find principal stresses and the principal planes for a rectangular steel bar subjected to a tensile stress of 100N/mm2 and shear stress of 50N/mm2. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State torsion equation and list its parameter. What are the assumptions made in theory of torsion? |  |
| b) | A solid circular shaft of 3m length and 200mm diameter is subjected to a torque of 100 KN-m. Find the angle of twist if modulus of rigidity is 1 x 105N/mm2. |  |
| c) | A hollow shaft 450mm external diameter and 250mm internal diameter is subjected to a 375KW at 105 rpm. Find the shear stress and angle of twist if length is 4500mm and modulus of rigidity is 8 x 104N/mm2. | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the ILD for simply supported beam of span L for reaction at left and right support at a section in ‘a’ from left support when unit load is moving from left to right. |  |
| b) | Two wheel loads 200N and 80N spaced 1m apart roll on the girder 20m in span. Find the maximum positive and negative shear force at a section 4m from left support |  |
| c) | A uniformly distributed load of 6KN per m run of length 5m moves on girder of span 20m. Find the maximum positive and negative shear force at a section 6m from left support. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | What are the uses of indeterminate structures? Explain with examples internal and external degree of indeterminacy of structure. | (6) |
|  | For students of Revised Scheme only: |  |
| b) | A thick steel cylinder having an internal diameter of 100mm and external diameter of 200mm is subjected to an internal pressure of 55N/mm2 and external pressure of 7N/mm2. Find the maximum and minimum hoop stress. | (6) |
|  | OR |  |
| b) | A thin spherical shell is of 1.2m diameter and shell thickness is 6mm. Determine the increase in diameter and increase in volume of the shell when an internal pressure of 1.2N/mm2 is applied. Take 1/m = 0.3 and E=2 x 105N/mm2. | (6) |
|  | For students of Old Scheme only: |  |
| b) | A flitched beam consists of two wooden joists 150mm wide and 300mm deep with a steel plate 250mm deep and 10mm thick placed symmetrically between them. If the stresses in steel and timber shall not exceed 140N/mm2 and 6N/mm2 respectively, find the moment of resistance of the section. Take modular ratio of steel timber as 20. | (6) |
|  | OR |  |
| b) | A flitched beam consists of two wooden joists 200mm wide and 300mm deep strengthened by a steel plate 12mm thick and 200mm deep one on the either side of the joist. If the maximum stress in wood is 6N/mm2, find the corresponding maximum stress in steel. Find also the moment of resistance of the section. Take ES=20EW. | (6) |

***********