## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Computer Engg.

Subject: Microprocessors-III(8086) [5178] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the purposes of the following pins of 8086: (i) INTR (ii) HLDA (iii) ALE |  |
| b) | How is the physical address generated in the 8086? |  |
| c) | Explain immediate addressing mode with example. |  |
| d) | Explain any two arithmetic instructions in 8086 with example. |  |
| e) | State and explain any three assembler directives. |  |
| f) | Draw a diagram showing interfacing of 8086 to memories. |  |
| g) | What are hardware interrupts and software interrupts in 8086? |  |
| h) | Explain the functions of any three 8087 instructions. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operating modes of 8086. |  |
| b) | Draw and explain the architecture of 8086 microprocessor. |  |
| c) | Explain relative addressing mode and implied addressing mode with example. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Illustrate with examples different type of shift and rotate instructions. |  |
| b) | What are conditional transfer instructions? Explain any four with examples. |  |
| c) | List and explain any three string handling instructions with examples. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an assembly language code in 8086 to explain IF-THEN-ELSE |  |
| b) | Write 8086 assembly language program to add 2 16-bit hexadecimal numbers. |  |
| c) | Explain Read Machine Cycle with a neat diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how interrupts are handled in 8086. |  |
| b) | State and explain the various 8087 data types. |  |
| c) | List the salient features of 80286 and 80386 processors. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Flag register in 8086 |  |
| b) | Procedures and Macros |  |
| c) | 8086 timing parameters |  |
| d) | Priorities of 8086 interrupts |  |

***********