## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Architectural Engg.

Subject: Building Construction-IV(AE402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Sketch 6 types of steel sections used in structural steel work. |  |
| b) | State three causes that lead to dampness in buildings. |  |
| c) | Name any three companies that manufacture waterproofing materials. |  |
| d) | Name three types of mortars used for plastering wall surfaces. |  |
| e) | Suggest three types of floor systems used in cost effective housing. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For an Automobile workshop, Architect has recommended steel structure. Elaborate on any four advantages of structural steel work over R.C.C. Also mention the demerits of such structure, if any. |  |
| b) | List the three methods of connecting steel sections. Elaborate on the types of fastenings used in the connections. Draw sketches of the fastenings. |  |
| c) | Explain with the help of neat sketches the following terms applicable in structural steel work: i) Latticing ii) Splicing. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat diagrams of any three types of steel trusses recommended for spans from 6m to 9m. Label the principal components in any one of the trusses. |  |
| b) | A roof of a theatre in spanned with steel trusses having clear span of 15 metres and are places at 4m c/c. choose an appropriate type of truss and draw a section showing roof covering, fixing details, details at wall supports, ridge joint. |  |
| c) | Explain with sketches the measures recommended for making steel structural members fire resistant. |  |
| Q.No.4. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | A client wants his proposed residence to be damp free. Suggest provisions in design an Architect would make while planning the residence. |  |
| b) | State any three considerations in construction of a cavity wall for it to be an effective method of damp prevention. |  |
| c) | Draw a neat-labelled section through a plinth to indicate position of DPC to protect flooring from dampness. |  |
| d) | State any two ill effects of exposing RCC slab to rains in Goa. |  |
| e) | Define brick bat coba. State all the materials used in it and draw a sketch to indicate position of brick bat coba as waterproofing at terrace level. |  |
| f) | Draw a neat labelled cross section through a toilet sink on second floor. Show the waterproofing treatment details in the cross-section. | ….2/- |
|  | -2- |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Define the following terms: i) Rendering coat ii) Peeling of plaster |  |
| b) | Briefly explain the procedure of providing ‘Sand faced plaster finish’ on a new external wall. |  |
| c) | An old externally painted wall requires to be cladded with ceramic tiles. Briefly explain the method of surface preparation. |  |
| d) | A timber floor is to be provided over room of width 3m. Suggest the type of floor. List the components required to assemble the floor. |  |
| e) | Suggest appropriate floor furnishing material for i) Workshop mezzanine floor on cement bonded particle board ii) A Dentist’s clinic on RCC floor iii) Bedroom planned on timber floor |  |
| f) | Draw a precast floor. State 2 advantages of precast floor. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Given a room admeasuring 8m x 6m. Plan a floor at 3m height above plinth. Draw the schematic plan on the floor and sketch necessary details required for execution of the same. |  |
| b) | Draw any two types in hollow tile ribbed floor. |  |
| c) | It is planned to connect two houses 4m apart by a steel fabricated passage 1.2m wide at 1st floor level. Make suitable assumptions and sketch a cross section to show assembly of steel sections. |  |
| d) | Define a ‘filler joist floor’. Draw a neat labelled cross section and explain the construction sequence. |  |

***********