## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC

Subject: Advanced Microcontrollers(EN613)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Which are the external hardware interrupts of 8051? What are their ISR addresses? |  |
| b) | What are the contents of register IE upon reset? What do these contents mean? |  |
| c) | What address in the interrupt vector table is assigned to serial interrupt? How to enable it? Also explain its action upon activation. |  |
| d) | What is baud rate? Which timer is used to set the baud rate? |  |
| e) | What is the significance of SMOD bit? Is it related to baud rate? How? |  |
| f) | What are the different hardware development tools? |  |
| g) | Draw interfacing diagram of LCD display to 8051. Show relevant pin connections. |  |
| h) | Explain in brief about PIC registers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an assembly language programme to generate two square waves of 5KHz at P1.3 & another of 25KHz at P2.3 using timer interrupts. Use timer in mode 2. Assume crystal frequency = 22MHz. |  |
| b) | Explain edge triggered & level triggered modes of external hardware interrupts. |  |
| c) | List the steps of generation of square wave of particular frequency on the part of 8051 using timer ‘0’ in mode ‘1’. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain SCON & PCON registers. |  |
| b) | Explain the steps to transmit the data serially by 8051, using serial interrupt. |  |
| c) | Write an assembly language program to transfer 10 data bytes from RAM 45H, serially. Assume crystal frequency = 11.0592MHz & baud rate 9600. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Rs-232 bus standard in detail. |  |
| b) | Compare all serial protocols. |  |
| c) | Explain compiler, debugger & simulator in brief. |  |
| d) | What is an embedded system? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw an interfacing diagram of stepper motor to 8051. Write a program to rotate it continuously clockwise in full step sequence. |  |
| b) | Draw an Interfacing diagram of four numbers of multiplexed 7-segment display alongwith the decoder. Write a programme to display digits ‘1’, ‘2’, ‘3’, ‘4’ on them respectively. Assume 1msec delay for displaying the digit. Crystal frequency = 11.0592MHz. -2- | ……2/- |
| c) | Draw interfacing diagram of DAC (0808) to 8051. Write a program to generate saw tooth waveform. Crystal frequency = 11.0592MHz. |  |
| d) | Draw interfacing diagram of 4x4 keyboards to 8051 & write a program to read the key pressed and store to accumulator. Assume your own ports for connection. Crystal frequency = 11.0592MHz. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain memory organization in PIC microcontrollers. |  |
| b) | Draw a functional block diagram of PIC microcontroller & explain. |  |
| c) | List special features of PIC microcontrollers. |  |

***********