## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(SH603) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Large angle stability |  |
| b) | Buoyancy curve |  |
| c) | Permeability |  |
| d) | Framing system in a ship |  |
| Q.No.2. | The following information has been obtained from the cross curves of stability of a ship of displacement 40,000 tonnes and KG =10m. | (12) |
| Angle of heel; 0; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN(m); 0; 0.9; 1.92; 3.11; 4.25; 6.3; 8.44; 9.39; 9.29; 8.5. |
|  | Determine i) Meta centric height ii) Dynamic stability at 15. |  |
| Q.No.3. | A box shaped vessel 75m long, 10m wide and 6m deep is floating in salt water on an even keel at a draft water 4.5m. Find the new draft if a forward compartment 5m long is bilged. KG = 2.5m. | (12) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Sketch and describe stiffened bulkhead. | (8) |
| b) | Draw a neat sketch to show how water tightness is maintained in pipe penetration and longitudinal structural member penetration of transverse bulkhead. | (4) |
| Q.No.5. | A box shaped vessel of uniform construction is 30m long and displaces 352 tonnes when empty. It is divided by transverse bulkheads into four equal compartments. Cargo is loaded into each compartment and level stowed as follows. No. 01 hold - 192 tonnes No. 02 hold - 224 tonnes No. 03 hold - 272 tonnes No. 04 hold - 176 tonnes Draw the shear force and bending moment diagram. | (12) |
| Q.No.6. | Answer the following Questions: |  |
| a) | With neat sketch explain the procedure to obtain floodable length curve. | (4) |
| b) | Explain the use of flexure formula in determining the section modulus of a transverse bulkhead stiffener. | (8) |

***********