## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(4104)/(EE301) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define Active, Reactive and Apparent power in AC circuit using power triangle. |  |
| b) | State maximum power transfer theorem and explain it briefly. |  |
| c) | Derive an expression for resonant frequency in a series circuit. |  |
| d) | Explain different types of transients. |  |
| e) | Draw De Saunty’s bridge circuit. Give its equation at balance. |  |
| f) | Define linear and bilinear circuit elements. Give example of each of them. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Apply Thevenin’s theorem to find current flowing through the 12Ω resistor in the circuit of fig.2(a). |  |
| b) | Using nodal voltage method find the magnitude and direction of current ‘I’ in the network of fig.2(b). |  |
| c) | A network of resistance is formed as shown in fig.2(c). AB=9Ω; BC=1Ω; CA=1.5Ω forming a delta and AD=6Ω; BD=4Ω; CD=3Ω forming a star. Compute the network resistance measured between A and B. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A coil having a resistance of 15 ohm and an inductance of 0.2H is connected in series with another coil having a resistance of 25Ω and an inductance of 0.04H to a 230V, 50Hz supply. Determine i) Voltage across each coil ii) Power dissipated in each coil iii) Power factor of the circuit as a whole. |  |
| b) | Two impedances Z1=(8+j6) and Z2=(3-j4) ohms are in parallel. If total current of the combination is 25A, find the current taken and power consumed by each branch. |  |
| c) | An ac series circuit has a resistance of 100Ω, an inductance of 0.2H and a capacitance of 60µF. Calculate i) the resonant frequency ii) the current and iii) Power at resonance given that the applied voltage is 200V. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Three impedances each of magnitude (15-j20) ohms are connected in mesh across a 3 phase 400 Volt ac supply. Determine the phase current, line current, active power and reactive power drawn from the supply. |  |
| b) | A 3 phase load consists of three similar inductive coils each of resistance 50Ω and inductance 0.3H. The supply is 415V, 50HZ. Calculate the line current, power factor and the total power when the load is star connected. |  |
| c) | Each phase of a delta connected load consists of a resistor of 50Ω and a capacitor of 50µF in series. Calculate i) the line and phase currents ii) the total power and iii) KVA when the load is connected to a 440V, 3, 50Hz supply. | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find out the positive, negative and zero phase sequence components of the following set of three unbalanced voltage vectors VA=1030 VB=30-60 VC=15145. |  |
| b) | Draw Hay’s bridge. Give their equation at balance and list their advantages and disadvantages. |  |
| c) | The four arms of a Wein parallel bridge ABCD supplied with a sinusoidal voltage have the following values: AB - 200 ohm resistance in parallel with 1µF capacitor, BC – 400 ohm resistance, CD – 1000 ohm resistance and DA - resistance R in series with 2µF capacitor. Determine i) the value of ‘R’ and ii) supply frequency at which the bridge will be balanced. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Show the graphical representation for variation of resistance, inductive reactance, capacitive reactance, net reactance, circuit impedance and current I0 with frequency for an ac series resonant circuit. |  |
| b) | When a voltage of 100V at 50Hz is applied to a choking coil A, the current taken is 8A and the power is 120W. When applied to coil B, the current is 10A and power is 500W. What current and power will be taken when 100V is applied to the two coils connected in series. |  |
| c) | For the circuit shown in fig Q.6(c), calculate the applied voltage, active power, reactive power and apparent power. Construct the power triangle. What is the p.f. of the circuit? |  |