## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/MED

Subject: Basic Electronics Engineering(CS308) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Differentiate between Zener diode & normal diode. |  |
| b) | Explain the need of using filter circuit at rectifier output. |  |
| c) | Calculate IE in a transistor for which  = 50 & IB 20µA. |  |
| d) | Define  &  in a transistor. |  |
| e) | Differentiate between JFET & BJT. |  |
| f) | Differentiate between ROM & RAM. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define ripple factor. With the help of neat circuit diagram & waveforms, explain the operation of the circuit which removes ripples from the rectified output waveform. |  |
| b) | Explain the construction, operation & characteristics of Zener diode. |  |
| c) | A Zener regulator circuit maintans 30V across 2KΩ load. If series resistance is 200Ω and zener current rating is 25mA, find the range of input voltage needed to ensure proper operation of zener voltage regulator. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction of NPN transistor in detail. Also draw symbol of NPN & PNP transistor. |  |
| b) | Explain output characteristics of a transistor in CE configuration & explain. |  |
| c) | Explain the concept of ICBO & ICEO in transistor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and operation of N-channel JFET. |  |
| b) | Explain the construction of N-channel depletion type MOSFET with neat diagram. Also, draw the symbol of the same. |  |
| c) | Explain the operation of N-channel enhancement type MOSFET. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of CMOS NAND gate with neat diagram. |  |
| b) | Compare TTL logic family and CMOS logic family. |  |
| c) | State the features & applications of PROM, EPROM & EEPROM. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Revere breakdown mechanism in Zener diode |  |
| b) | Advantages of CE configuration of transistor over other configurations |  |
| c) | P channel depletion type MOSFET |  |
| d) | Flash memories |  |

***********