## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(4242)/(FE602) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the purpose of estimating? |  |
| b) | State the functions of estimating department |  |
| c) | Explain the factors affecting welding cost. |  |
| d) | Explain different sheet metal operations. |  |
| e) | Define the following terms in Estimation: i) Detailed Estimate ii) Rough Estimate iii) Supplementary Estimate. |  |
| f) | State the advantages and disadvantages of Lumpsum contract. |  |
| g) | State the importance of Measurement page. |  |
| Q.No.2. | Prepare Measurement sheet and Abstract sheet for a “Roof Truss Structure” used for industrial shed of span 10m and 36m long with spacing of trusses at 3m centre to centre for the following items: i) Structural steel in trusses and purlins ii) AC roof sheet coverings. Refer Fig.1. Assume suitable market rates. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Calculate the time required to machine the job shown in fig.2 of mild steel from a round stock of 60mm diameter of length 100mm. Assume suitable machining parameters. | (8) |
| b) | What are allowances? Explain any three allowances, which are considered while determining machining time. | (4) |
|  | ### OR |  |
| b) | How is estimation of sheet metal operation time done. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Estimate the cost of material of the job shown in fig.2 made up of cast iron costing Rs.280/Kg and having specific weight of 7.2 gms/cc. |  |
| b) | Calculate the energy charges of a diesel engine driven welding generator having an engine of 4BHP. The output arc current was 110 Amps and arc voltage of 22 Volts when used for 5 hours. The engine consumes 60ml of diesel per BHP and the cost of the diesel is Rs.58 per litre. -2- | ……2/- |
| c) | Draw neat sketch of Break even chart. Explain any one method of carrying out replacement studies. |  |
| d) | Explain the various elements of costing. (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A lathe machine worth Rs.6,00,000/- was purchased 3 years before with a useful life of 12 years and scrap value of Rs.1,00,000/-. Calculate the depreciation amount today if the installation cost of Rs.15,000/- and demolition cost of Rs.10,000/- is to be considered. |  |
| b) | Briefly explain the following: i) Breach of contract ii) Security Deposit. |  |
| c) | Differentiate between the following: i) Item rate contract and Percentage rate contract ii) Labour contract and Materials supply contract |  |
| d) | A Press costs Rs.8,00,000/- and delivers four number of useful strokes per minute. The press works on a 6 HP motor and electric power charge is Rs.25/unit. Calculate the cost of single stroke of press. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain i) the requirements of valid contract ii) Schedule of rates iii) Work charged establishment. |  |
| b) | Briefly explain settlement of dispute in Engineering contract by Arbitration. |  |
| c) | Explain procedure involved in notices inviting tender and submission of tenders with a suitable civil engineering work example. |  |
| d) | Explain the concept of costing. State the various methods of costing. (only for students of old scheme) |  |