## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036)/(ME614) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the function of expansion valve in the car air conditioning system? |  |
| b) | State the functions of a chassis frame. |  |
| c) | Distinguish between throttle body injection and multi-point injection. |  |
| d) | What happens if clutch pedal free play is excessive? |  |
| e) | Why should the battery be not overcharged? |  |
| f) | Why is unspring weight kept as low as possible? |  |
| g) | What is a combination spanner? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain how the power flows from the engine to the wheels in a 2-wheel drive vehicle. Also lable the wheel base and wheel track on the same sketch. |  |
| b) | Discuss the advantages and disadvantages of framed and frameless chassis. |  |
| c) | With the help of a neat labelled sketch explain the construction and working of the steering linkage for a rigid axle suspension vehicle. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the advantages of multipoint injection over carburetion engine. |  |
| b) | With the help of a neat block system, explain the engine management system of an Electronic Fuel Injection petrol engine. |  |
| c) | With the help of a neat sketch, explain a sealed beam headlight unit. |  |
| d) | With the help of a neat sketch, explain the construction and working of a lead-acid battery. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the fluid flywheel characteristics with the help of a neat sketch showing % slip v/s engine speed. Hence, explain how should be the driver’s control. |  |
| b) | With the help of a neat sketch explain the working of a sliding mesh gear box. |  |
| c) | What is double declutching? Is it required to be done in a synchromesh gearbox? Explain, why? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the layout of the refrigeration system for car air-conditioning. |  |
| b) | Explain the use of the following tools in automotive maintenance: i) Stroboscope ii) Torque wrench. -2- | …..2/- |
| c) | With the help of neat sketch, explain the working of pneumatic brake system in an automobile. (only for students of revised scheme) |  |
| c) | Explain articles 43 and 44 of motor Vehicle Act 1988. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch, explain the working of recirculating ball type steering gerbox. |  |
| b) | With the help of a neat sketch, explain the working of an integral type hydraulic power steering system. |  |
| c) | With the help of a neat sketch, explain the construction and working of McPherson strut type independent suspension. |  |

***********