## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI

Subject: 8051 Microcontroller(EN501)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any two differences between microcontroller and microprocessor. |  |
| b) | What is a data memory? What is the address range of data memory available to programmer in 8051? |  |
| c) | What is DPTR? Write instruction and its meaning using DPTR. |  |
| d) | Write contents of accumulator, after execution of following instructions: MOV A, #55H RR A |  |
| e) | What is an interrupt? Name different interrupts of 8051. |  |
| f) | Write instruction to copy data 87H to register R0 of Bank 1 of internal RAM. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw PSW and explain each flag of PSW. |  |
| b) | What is a port? Draw circuit diagram of any one of the parallel ports & explain reading operation. |  |
| c) | Write a short note on stack and stack pointer. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw TMOD register. Write instruction to select Timer 0 in Mode 2. Explain Mode 2 of timer mode. |  |
| b) | What is a baud rate? Explain SBUF and transmission of serial data. |  |
| c) | Draw and explain bit addressable RAM of 8051. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the instruction to select Mode 1 for serial transmission. Explain Mode 1 of serial transmission. |  |
| b) | State different addressing modes. Identify addressing modes for following instructions. - MOV R1, #40H - MOV A, @R1 - MOV 40H, #40H - MOV @RO, #15H |  |
| c) | Write an assembly language programme to find number of O’s in the data byte AFH. Display result at location 40H of external RAM. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write output of each of following instructions and hence output of programme. MOV A, #57H SWAP A ADD A, #17H DA A -2- | ……2/- |
| b) | Write an assembly language programme to transfer five data bytes from external RAM locations starting from 50H onwards to internal RAM locations staring from 35H onwards in reverse order. |  |
| c) | Write an assembly language programme to find number of times data byte ABH appears in a block of 10 data bytes starting from 32H onwards of internal RAM. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain following instructions with example: - CJNE A, add, radd - JNC radd |  |
| b) | Write a short note on subroutine. |  |
| c) | Write a programme to divide 40H by 06H and save the result at location 33H and remainder at 34H. |  |
| d) | Write short note on – Programme range for jump instructions. |  |

***********