## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/MED/COMP/E&EE

Subject: Digital Electronics-II(4138) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Add the following BCD numbers - 0111, 0101 - 0001, 0110 |  |
| b) | What is a multiplexer? Draw the circuit of a 2 to 1 multiplexer. |  |
| c) | What advantage does the continuous type A/D converter have over the counter type A/D converter? |  |
| d) | Define propagation delay. Determine the clock cycle time for a system that uses a 8 MHz clock. |  |
| e) | What is matrix addressing? List its advantage. |  |
| f) | List an advantage and an application of FPGA. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit and write the truth table for: - Half adder - Half subtractor |  |
| b) | Perform the following: - 10112 x 11012 - Divide 1101012 by 1112 |  |
| c) | Draw the logic diagram of a 4-bit adder-subtractor and explain its operation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a demultiplexer? Draw the logic circuit of a 1 to 8 demultiplexer? |  |
| b) | With a neat logic diagram explain a Decimal to BCD encoder. |  |
| c) | Draw the logic diagram and explain the operation of a 1 of 8 decoder. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of a 3-bit resistive divider. Determine the output for a digital input of 101. Assume 0=0V and 1=+10V List 2 drawbacks of the resistive divider. |  |
| b) | With a neat block diagram explain the successive approximation technique of analog to digital conversion. |  |
| c) | Draw a neat block diagram of a continuous A/D converter and explain how an analog signal is converted to a digital signal. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and the explain the operation of an astable multivibrator using IC 555. Write the expressions for frequency of oscillation and duty cycle. |  |
| b) | Draw a logic diagram of a 4096 x 8 ROM showing address lines, data lines and chip select lines. |  |
| c) | Differentiate between SRAM and DRAM. Construct a 256x4 RAM using 256x1 memory chips. |  |
|  | -2- | ……2.- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is contact bounce? Draw the circuit of a debounce circuit and explain how the contact bounce problem is eliminated. |  |
| b) | What is a PLA? Construct a PLA having 3 input variables and 3 output variables. |  |
| c) | Illustrate ROM as a PLD to produce the following outputs: Y0=ABC+ABC+ABC Y1=ABC+ABC Y2=ABC |  |

***********