## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-I(EE302) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With reference to a transformer, define: (i) Step up transformer (ii) Primary winding. |  |
| b) | List parts of DC machine. (any 6) |  |
| c) | State requirements of DC welding generator. |  |
| d) | What is back emf? State its significance. |  |
| e) | State salient features of distribution transformer. |  |
| f) | State conditions to be satisfied before connecting two transformers in parallel. |  |
| g) | State any two advantages and one disadvantage of an Auto-transformer. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of diagram explain the working of simple loop D.C. generator. |  |
| b) | With the help of characteristics explain the process of voltage built up in DC shunt generator. |  |
| c) | Give classification of DC generators. Draw their connection diagram. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain Torque v/s Speed characteristics of DC series motor. Also, explain why such motor should not be operated on No load. |  |
| b) | With a neat diagram, explain four point starter. |  |
| c) | Briefly explain speed control methods for DC series motor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of diagram distinguish between core type and shell type transformer. |  |
| b) | What is a tap changer? Why it is needed? State difference between OFF line and ON line tap changer. |  |
| c) | An ideal 25KVA transformer has 500 turns on the primary winding and 40 turns on the secondary winding. The primary is connected to 3000V, 50 Hz supply. Calculate (i) Secondary emf (ii) Primary full load current (iii) maximum value of flux in the core. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain phasor diagram of transformer supplying rated current for an inductive load. |  |
| b) | A 10KVA, 2000/400V single phase transformer has R1 = 5Ω, X1 = 12Ω, R2 = 0.2Ω and X2 = 0.48Ω. Determine the equivalent impedance of the transformer referred to (i) Primary side (ii) Secondary side. |  |
| c) | For a 50KVA transformer wattmeter reading for O.C. test at rated voltage was 500W. For S.C. test when rated current was flowing wattmeter reading was 800W. Determine the efficiency of transformer at (i) full load and 0.8 p.f. lagging (ii) half full load and 0.8 p.f. lagging. |  |
|  | -2- | ……2/- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain procedure of connecting two three phase transformers in parallel. |  |
| b) | Write short note on welding transformer. |  |
| c) | Write a short note on furnace transformer. |  |

***********