## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE/COMP/MED

Subject: Digital Electronics(CS309) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following: - 92.05(10) = ___________________ (BCD) - 0110(10) = _________________ (2) - 10110.101(2) = ________________ (16) |  |
| b) | Convert 35(10) to Gray. |  |
| c) | Draw X-NOR gate. Write down the truth table and Boolean equation for the output. |  |
| d) | Write the statements and Boolean equations for De Morgans laws. |  |
| e) | Draw J-K FF and write down truth table. |  |
| f) | What is a Counter? What do you understand by Mod 10 Counter? |  |
| g) | Draw 4 bit serial in serial out shift register. |  |
| h) | What is a Demultiplexer? Draw logic diagram of 1 to 4 Demultiplexer. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw logic diagram of Half subtractor. Write down the truth table and Boolean equations for outputs. |  |
| b) | Simply following using K-Map. Y=ABC+ABC+ABC+ABC |  |
| c) | What is the difference between Decoder and Demultiplexer? Draw logic diagram of 2 to 4 Decoder. |  |
| d) | Implement AND, OR, NOT using NOR. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain clocked R-S FF using NAND, with truth table. |  |
| b) | Draw T FF. Write down its truth table and explain Master Slave FF. |  |
| c) | Perform the following using 2’s complement method. - 20(10) – 30(10) - -20(10) + 30(10) - -20(10) – 30(10) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a register? What are different types of shift registers? Explain any one of them with 4 bits. |  |
| b) | Draw 4-bits synchronous up counter. Draw timing diagram. Write down the count. |  |
| c) | Write any three points of differentiation between Synchronous and Asynchronous counters. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State different types of Digital to Analog converters. Explain binary ladder ADC. -2- | …..2/- |
| b) | State different types of Analog to Digital converters. Explain Successive Approximation ADC with example. |  |
| c) | Write short notes on: i) Duality theorem ii) ASCII Code |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write the truth tables for AND, NOR gates. Write the Boolean equation for following output Y. |  |
| b) | Write a short note on – Analog and Digital signals. |  |
| c) | Write a short note on Decade Counter. |  |
| d) | Write the output of following Boolean equations: - A+A = - A⋅A = - A+A = - A+0 = |  |

***********