## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg.

Subject: Design of Structures-II(Steel) [CE602) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) IS code 800-2007 & IS 875-part 3 1987 are permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Briefly describe the types of loads on steel structures and write any 3 applications of structural steel. |  |
| b) | What are the different types of welded connections? Draw neatly symbols of fillet and butt welds. |  |
| c) | Classify and explain briefly the modes of failure in Tension members. |  |
| d) | With the help of neat sketch explain the phenomenon of web crippling in a beam. |  |
| e) | Draw a neat sketch of column base and explain the steps involved for determining size of base plate. |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Design a tension member to carry a pull of 850KN. The member is 3.2m c/c of intersections. Design the member using channel section. | (9) |
| b) | An ISLC 300 @ 324.7 N/m (Fe 410 grade of steel) is to carry a factored tensile force of 810KN. The channel section is welded at the site to a gusset plate 12mm thick. Design a fillet weld, if the overlap is limited to 350mm. OR | (6) |
| c) | A tie member consisting of ISA 150 x 115 x 12mm (Fe 410 grade steel) is welded to 12mm thick gusset plate at site. Design welds to transmit load equal to the design strength of the member. | (6) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Design a double angle discontinuous strut to carry a load of 240KN. The length of strut is 3.8m. Design the end connections using fillet weld. Take fy = 250MPa. | (9) |
| b) | Determine factored strength of a strut 4m long, consisting of 2 ISA 150 x 150 x 10 connected on either side of gusset plate 12mm thick by welding. OR | (6) |
| c) | Design a slab base for a built up column consisting of two channels (ISLC350) back to back separated by a distance of 215mm and carrying a factored load of 1750KN. Take M15 grade concrete, Fe 410 grade steel and safe bearing capacity of soil as 340 KN/m2. | (6) |
| Q.No.4. | Answer the following Questions: |  |
| a) | A steel floor beam is a building has a span of 6.5m. It is simply supported over supports and carries a uniformly distributed load of 40KN/m inclusive of self weight. Design the beam considering it as laterally supported beam. | (9) |
| b) | Calculate moment, carrying capacity and shear strength of a laterally supported ISMB 300 @ 433.6 N/m beam member having span of 4m. Take fy=250 MPa. OR | (6) |
| c) | Explain the web crippling and web buckling in a beam section. | (6) …..2/- |
|  | -2- |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Design an angle iron purlin for an industrial building to support a galvanized iron sheet roof. Following data to be used in design: spacing of truss c/c=6m, span of truss=10m, spacing of purlins c/c=1.5m, inclination of rafter with horizontal=30º, Intensity of wind pressure =2KN/m2, Weight of GI sheets =130N/m2 , fy=250 MPa. | (9) |
| b) | Calculate the dead and live load on a fink type roof truss for the following data and mark the loads on the nodes of the truss. Span =12m, pitch = 1/4 of span Height of eves level=10m from the ground Spacing of truss = 4.5m c/c Assume any other required data. | (6) |

***********