## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Transportation Engineering-I(4006) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the classification of roads according to the materials of construction. |  |
| b) | Why is it necessary to stabilize earth roads? |  |
| c) | Define i) Tar ii) Cut back iii) Seal coat. |  |
| d) | What are the merits of concrete roads? |  |
| e) | Explain the terms: i) Skew bridges and ii) square bridges. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail the various types of transportation systems employed in India. |  |
| b) | Why are curves provided in Roads? Explain different types of curves with sketches. |  |
| c) | Draw a neat cross section of a WBM road and name its various parts. Also state the dimensions of these components. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write the construction procedure of bituminous concrete road in detail. |  |
| b) | What are the different defects that occur in concrete roads? What are the remedial measures taken to rectify these defects? |  |
| c) | Explain continuous bay method of construction of concrete roads. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the following terms with respect to hill roads: i) Protective works ii) Catch water drains iii) Floating gradient. |  |
| b) | How is surface and sub surface drainage done in hill roads? |  |
| c) | Define the following terms: i) Scour ii) Afflux iii) Economic span. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of a suspension bridge and name its component parts. Where are these bridges provided? |  |
| b) | What is cause way? Explain different types of causeways. |  |
| c) | Write short notes on: i) Repair and maintenance of culverts ii) Types of Gauges in Railways. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Harbour and its function |  |
| b) | Freeboard |  |
| c) | Bearings in bridges |  |
| d) | Super elevation |  |

***********