## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(FE404) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the materials used for making Pig iron? Give their functions. |  |
| b) | How does high speed steel satisfy the service requirement of tool steel? |  |
| c) | State the effect of water, oil and air as quenching media on the structure of steel. |  |
| d) | What is the need for stress relief heat treatment of weldments? |  |
| e) | What are the advantages of direct electric arc furnace process of making steel? |  |
| f) | What do you understand by polymerization? Name some polymer additives. |  |
| g) | State the requirement for steel coating. Mention any three types of coating done on steel. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify steel on the basis of carbon percentage. Discuss the properties and applications of plain carbon steel. |  |
| b) | Explain Substitutional and Interstitial solid solutions with neat sketches. |  |
| c) | What is stainless steel? Gives the properties and uses of any two typing stainless steels. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concepts of Epitaxial growth and competitive growth in weld metal solidification. |  |
| b) | With the help of neat sketch explain TTT curves for plan carbon steel obtained during isothermal transformation. |  |
| c) | Explain the treatment necessary to produce Spheroidal Graphite cast iron. Give its two uses. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe Age hardening treatment in Al-Zn system with a neat sketch. |  |
| b) | Explain strain hardening of steel. State different methods of strain hardening with their uses. |  |
| c) | What is ‘Tempering of steel’? Explain Martempering and Austempering process with neat sketches. |  |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Differentiate between white and grey cast iron. |  |
| b) | Explain the process of extrusion with simple sketch. What are its applications? |  |
| c) | Explain the principle phenomenon used in cathodic protection of steel. |  |
| d) | Write a short note on shot blasting and sand blasting of steel. |  |
|  | -2- | …….2/- |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Weld cracks |  |
| b) | Vibratory stress-relief of weldment |  |
| c) | Alloy structural steel |  |
| d) | Bessemer process of steel making |  |
| e) | Composite materials |  |

***********