## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Common

Subject: Elements of Electrical Engg.(3001)/Basic Electrical Engg.(CS304)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Efficiency of transformer ii) Voltage regulation of a transformer. |  |
| b) | Explain the necessity of earthing in Electrical installation. |  |
| c) | Briefly explain the working principle of DC motor. |  |
| d) | How can you reverse the direction of rotation of a 415V, 10HP, 3 phase delta connected Induction motor. |  |
| e) | Explain the function of fuse in an electrical circuit. |  |
|  | only for students of revised scheme: |  |
| f) | Briefly explain the concept of phase sequence in 3 AC system. |  |
| g) | Draw the power triangle for an AC system and define: i) Real power ii) Apparent power |  |
|  | only for students of old scheme: |  |
| f) | List out the various tests to be carried out on a wiring installation. |  |
| g) | State any three IE rules relevant to domestic wiring. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat labelled diagram explain Plate earthing. |  |
| b) | List down the various methods of reducing earth resistance. |  |
| c) | Compare Star and Delta connections of 3 phase AC system. students of revised scheme) | (only for |
| c) | List down the factors affecting the current carrying capacity of a cable. (only for students of old scheme) |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the principle of operation of a transformer. |  |
| b) | Explain back emf in a dc motor. What is its significance in the operation of dc motor? |  |
| c) | For a sinusoidal AC voltage define the following: - i) RMS value ii) Form factor iii) Peak factor - iv) Average value (only for students of revised scheme) |  |
|  | only for students of old scheme: |  |
| c) | Compare underground distribution system with over head distribution system. |  |
| d) | State the various types of wiring systems and compare them with respect to accessibility, life mechanical protection & scope for future load extension. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the comparison between Squirrel cage Induction motor and Slip ring Induction motor. |  |
| b) | Write a short note on ELCB. |  |
| c) | With a neat diagram explain construction and operation of any one type of single phase Induction motor. |  |
| d) | Explain any one type of test performed on a wiring installation that requires use of a Meggar. (only for students of old scheme) | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagrams list the types of DC motors based upon connection of field and armature winding. |  |
| b) | Differentiate between HRC fuse and rewirable fuse. |  |
| c) | Wirth a neat sketch explain the principle of operation of an Universal motor. |  |
| d) | With the help of a neat diagram explain the general construction of a power cable and function of its various layers. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a short note on MCB. |  |
| b) | With the help of a neat diagram explain the construction and working of any one type of starter used for starting 3 Squirrel cage Induction motor. |  |
| c) | With a neat diagram explain the construction and working principle of three phase Induction motor. |  |
| d) | Write a short note on power losses in a transformer. |  |

***********