## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Applied Electronics(4213) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the expression for voltage gain, current gain and power gain |  |
| b) | Draw the frequency response of a RC coupled amplifier. |  |
| c) | What are power amplifiers? List the different types. |  |
| d) | Explain positive feedback and negative feedback in amplifiers. |  |
| e) | Define CMRR. Also state its expression. |  |
| f) | Draw a differential amplifier with constant current source. |  |
| g) | State 3 advantage of using SSB technique. |  |
| h) | Explain the concept of virtual ground with respect to op-amps. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the hybrid equivalent circuit of CE amplifier with neat diagram. |  |
| b) | With reference to a transistor, define all the hybrid parameters. State expressions for hybrid parameters for transistor in CE configuration. |  |
| c) | With a neat diagram, explain the operation of a Direct coupled amplifier. Also state its advantages. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the frequency response of a transformer coupled amplifier and explain in detail |  |
| b) | With a neat diagram, explain the working of a class C power amplifier. |  |
| c) | Explain the working of a Hartley oscillator with necessary circuit diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the use of Positive feedback in oscillators. What are the requirements of oscillations? Also state Barkhausen’s criteria for sustained oscillations. |  |
| b) | Draw the circuit for voltage series using BJT and explain its working. |  |
| c) | Explain the process of AM Demodulation using a simple Diode detector circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat circuit and waveforms, explain generation of AM signal. |  |
| b) | Explain how a FM signal is demodulated using a Balanced slope detector. Draw necessary diagram. |  |
| c) | State the characteristics of an Ideal op-amp. Also state 3 advantages of op-amps. |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Op-amp as Non Inverting amplifier |  |
| b) | Op-amp as Differentiator |  |
| c) | Op-amp as Band Stop Filter |  |
| d) | Op-amp as Logarithmic Amplifier |  |

***********