## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Metrology & Q.C.(4047)/Engineering Metrology & Q.C.(ME406)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify the following designations of fits into the 3 types generally used: - H11- c11 ii) H7-s6 iii) H7-k6 |  |
| b) | What are ‘attributes’ and ‘variables’? Give examples. |  |
| c) | Draw a sketch of one involute gear tooth and label its various parts. |  |
| d) | Write a brief note on angle gauges and their uses. |  |
| e) | What care is necessary in using length bars? Why? |  |
| f) | Differentiate between ‘accuracy’ and ‘precision’. |  |
| g) | Give 3 applications where a dial indicator type comparator having range 0-10mm can be used. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is ‘wringing’ of slip gauges? Why is it necessary? |  |
| b) | Describe with a neat sketch, the construction and use of Angle Dekkor. |  |
| c) | Calculate the various sizes involved in the assembly 25H7f8, given that: - ‘i’ in microns = 0.453D+0.001D where D is mean diameter in mm - the upper deviation for shaft ‘f’ = −5.5D0.41 - 25mm falls in the diameter step of 18 to 30 mm |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Enlist the uses of comparators and advantages of Mechanical comparators. |  |
| b) | Explain ‘Chordal Thickness Method’ for measurement of gear tooth thickness. |  |
| c) | What are the pitch errors? What are the causes and effects of pitch errors? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the procedure to test the following: - Parallelism of lathe bed guideways - Drill machine axis perpendicularity to table - Squareness of a rectangular block |  |
| b) | Explain the following terms used in surface finish measurement: - Lay ii) Ra value iii) Sampling length |  |
| c) | Explain the method of implementation of Quality control in industries. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the procedure adopted to plot X & Rcharts? -2- | …..2/- |
| b) | A large number of samples were inspected and the data is summarised as below: Sample No.; Total; Average X; Range R. 1; 24.3; 6.1; 3.2. 2; 20.6; 5.2; 3.4. 3; 19.2; 4.8; 2.2. 4; 21.7; 5.4; 1.6. 5; 15.6; 3.9; 1.8. 6; 21.1; 5.3; 2.4. 7; 20.8; 5.2; 2.4. 8; 17.2; 4.3; 2.9. 9; 16.5; 4.1; 2.3. 10; 17.1; 4.3; 1.6. 11; 15.9; 4.0; 1.3. 12; 21.4; 5.4; 3.0. 13; 14.3; 3.6; 1.4. Use the following constants and compute the lower and upper limits for both X and Rcharts: A2 =0.729 D3 =0 D4= 2.282 Sketch the charts. |  |
| c) | Write an explanatory note on purpose of implementation of Quality Control. |  |
| Q.No.6. | Write short notes any four: | 4 x 3 = 12 |
| a) | OC curves |  |
| b) | Use of Autocollimator in straightness measurement |  |
| c) | Working of ‘Dial Indicator’ comparator |  |
| d) | Taylor’s principle of gauge design |  |
| e) | Need of inspection in Industries |  |
| f) | Types of ‘Go’ & ‘NOT-GO’ gauges |  |

***********