## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Production/Automobile Engg.

Subject: Machine Tools & Production Proc.-II(4060)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the uses of following devices used in Shaper (i) T-bolts and Clamps (ii) V-block vice (iii) Parallels. |  |
| b) | Illustrate with examples the types of jobs that can be done on a Planning machine. |  |
| c) | Give the general specifications of a Universal milling machine. |  |
| d) | What are the functions of a dividing head? |  |
| e) | State the difficulties encountered in deep hole drilling and how they are overcome. |  |
| f) | State the general safety precautions to be observed on a boring machine. |  |
| g) | Describe the main part of a slotting machine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch any mechanism used to obtain Quick return motion in a Shaper. Compare hydraulic drive with other methods of driving Shaper. |  |
| b) | How do you specify a Shaper? Explain the terms Cutting speed, Feed and Depth of cut in the context of a Shaper. |  |
| c) | Calculate the machining time for drilling 16 mm diameter 4 holes on a flange using the following data. Flange thickness – 30 mm, cutting speed - 220 rpm and feed - 0.2 mm/rev. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Under what circumstances is a radial drill used? Sketch a radial drill and label its parts. |  |
| b) | Compare boring operation with reaming operation. Also sketch & explain spot facing and trepanning. |  |
| c) | With a neat labelled sketch, explain the various parts of a Jig boring machine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why is an index plate used in a Slotter? State two points of difference between a Shaper and a Slotter. |  |
| b) | Explain with a neat sketch the working of open and cross belt drive of a Planer. |  |
| c) | How are Planning machines classified? Explain the terms (i) Clapper box (ii) Cross rail. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain - Straddle milling - Gear cutting operation on a milling machine. |  |
| b) | In what way does the milling operation differ from the planning operation? State chief features of Column & Knee type Universal milling machine. |  |
| c) | Sketch four tools on a Planner and give illustrations for which they are most suited. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Classification of milling cutters |  |
| b) | Upmilling and down milling |  |
| c) | Use of rotary table on a milling machine |  |
| d) | Arbor assembly |  |
| e) | General specifications of universal milling Machine. |  |

***********