## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication of Boilers & Pressure Vessels(5341)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is crowning of Rolls in Roll forming? |  |
| b) | What are form blocks (used in bending of tubes)? |  |
| c) | What are the advantages of manual spinning? |  |
| d) | Explain one method of compensating for Spring Back. |  |
| e) | What are the main reasons for stiffening sheet metal? |  |
| f) | Why is the design of a pressure vessel based on Hoop stress? |  |
| g) | What is earthquake load on a pressure vessel? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare the features of Hydraulic and Mechanical Presses. |  |
| b) | What are the hazards presented by press brakes? What are the safety aspects of press brakes? |  |
| c) | Explain any two Press Braking tools. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the characteristics of: |  |
|  | i) 3 roll type Pyramid Roll Bending machine. |  |
|  | ii)Roll down type Roll Bending machine. |  |
| b) | Explain the rigid methods of bending tubing with the help of mandrels. |  |
| c) | i)Discuss spinning speeds. |  |
|  | ii) Explain the IBR code briefly. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Stress-Strain diagram for mild steel. |  |
| b) | i) Explain one methods of wiring the edges of sheet metal for stiffness. |  |
|  | ii) What are the common service conditions to which pressure vessels are exposed? |  |
| c) | What is meant by longitudinal stress in a Thin Cylindrical Shell? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain what are the Primary Stresses in a pressure vessel. |  |
| b) | Explain two design precautions to tackle pressure vessel failures. |  |
| c) | i) What is a Hydro test? |  |
|  | ii) How can you prevent buckling of a pressure vessel under external load? |  |
| Q.No.6. | Write a short notes on any four: | 4 x 3 = 12 |
| a) | Advantages of Hot Bending of tubes over Cold Bending |  |
| b) | Thermal Stresses |  |
| c) | Selection of steel for pressure vessels exposed to Hydrogen |  |
| d) | Ligament efficiency |  |
| e) | Interpretation of welding Radiographs |  |
| f) | Dead load on pressure vessels |  |

***********