## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT

Subject: Construction-II(4009)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State the objects of foundations. Briefly explain methods of ground examination. |  |
| b) | Briefly explain the precautions to be taken in timbering of trenches. |  |
| c) | What are plinth beams? Where they are used? Explain briefly with help of sketch. |  |
| d) | With help of neat sketch, explain construction of box caissons. |  |
| e) | Explain briefly how the provisions are made for light, ventilation and dampness prevention in case of basements. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain briefly – (i) Freezing process (ii) Well point system. |  |
| b) | Explain briefly the necessity and construction details of grillage foundations. |  |
| c) | Explain with help of a neat sketch foundations used for individual columns and masonry walls. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Discuss briefly continuous footings, with help of neat sketch. |  |
| b) | State different equipments used in pile driving process, giving function of each of the equipment. Draw a neat sketch of standard steel pile frame. |  |
| c) | Differentiate between precast and cast–in–situ concrete piles. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Briefly explain: (i) Cased piles (ii) Un-cased piles (iii) Friction piles. |  |
| b) | What are well foundations? With neat sketch, explain the components of well foundations. Briefly explain the process of sinking of well foundations. |  |
| c) | Discuss briefly the causes and effects of dampness. State different materials used for prevention of dampness. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What is under pinning? State different methods of underpinning. Explain pile method with aid of neat sketch. |  |
| b) | Discuss the different types of portion materials used in building constructions. |  |
| c) | Write a brief note on retaining walls. |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Batching plant |  |
| b) | Compressor and pneumatic tools |  |
| c) | Hoists and pulleys |  |
| d) | Graders and dumpers |  |
| e) | Foundations on reclaimed soils |  |

***********