## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Repair Engineering(5334)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Preparation by ship staff for dry docking a ship |  |
| b) | Various methods of carrying out temporary repairs to a cracked ship side plate |  |
| c) | Precautions to be taken before entering a tank |  |
| d) | Measurement of Propeller drop and Rudder drop |  |
| e) | Checking water tightness of cargohold covers |  |
| f) | Painting scheme of ship’s hull |  |
| g) | Calibrating a main engine cylinder liner |  |
| h) | Testing of Co2 fire fighting system with compressed air |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Describe various services provided by the shipyard to a ship in the dry dock. |  |
| b) | What preparations a shipyard must make before taking a ship in the dry dock? |  |
| Q.No.3. | A steel plate measuring 4x2 mtrs has to be gas cut from the hull of a ship and an insert plate is to be fitted. With the help of a neat sketch describe the sequence of gas cutting and welding. Also describe the procedure for fitting the plate. ### OR | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | A double bottom tank used for storing fuel oil requires hot work to be carried out in the tank. Describe the process to be followed before hot work can be carried out. | (6) |
| b) | How the tank is pressure tested after repairs are completed? | (6) |
| Q.No.4. | Answer the following Questions: |  |
| a) | What checks are made on the propeller in a dry dock? | (4) |
| b) | Describe the procedure for removing and fitting a propeller. | (8) |
| Q.No.5. | Answer the following Questions: |  |
| a) | What checks are made on the Piston Rings of a large marine diesel engine? How will you decide whether the piston ring is to be changed? | (6) |
| b) | How is the cylinder liner calibrated? OR | (6) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Describe the procedure for removing the Piston of a main diesel engine. | (8) |
| b) | What checks are made on the Piston? | (4) |
| Q.No.6. | Answer any one of the following Questions: | (12) |
| a) | A centrifugal pump has been opened for overhaul. What checks will you make on various parts of the pump? |  |
| b) | Describe the procedure for testing a heat exchanger, of shell and tube type, for leaky tubes. |  |

***********