## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Civil Engg./CT

Subject: Design of Structures-II(RCC)[4016]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS-456-2000 code is permissible.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between “working stress” and “Limit state” method of design. |  |
| b) | Explain the following: (i)Balanced (ii) Under reinforced (iii)Over reinforced sections. |  |
| c) | What is partial safety factor? State the values used for loads and materials in Limit State method. |  |
| d) | Sketch the reinforcement details at beam and column junction. |  |
| e) | Compare simply supported and restrained slabs. |  |
| f) | State the necessity of providing cover to reinforcement. Mention the minimum covers for slabs, beams and columns. |  |
| g) | Sketch reinforcement details provided at edge beams. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Design a rectangular beam to carry an ultimate load of 25KN/m over an effective span of 4m. M25 and Fe415 are grade of concrete and steel available. |  |
| b) | A beam 230 mm x 450mm (overall) consists of 4#12mm as tensile reinforcement. The effective span of the beam is 5m. Determine the service load the beam can carry over the entire span. Take M20 concrete and Fe415 steel. |  |
| c) | An RC beam 250mm x 500 mm is reinforced with 4#16 mm as tensile reinforcement. Assuming M20 concrete and Fe415 steel, calculate depth of N.A, lever arm and moment of resistance of beam. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | An RC beam 250mm x 450mm (effective depth) consists of 5#12 as tensile reinforcement. If 2 out of 5 bars are bent up at 45 near the support, design the shear reinforcement for the beam to resist the ultimate shear of 200KN. Take the contribution of bent up bars as per IS requirements. M25 and Fe500 are concrete and steel respectively. |  |
| b) | Explain the various mechanisms involved in an RC beam to resist the shear force. |  |
| c) | A simply supported beam is 250mm x 400 mm (effective) carries an udl of 30 KN/m at working condition over an effective span of 4.5m. The beam is provided with 4#16mm. Design the shear reinforcement using M20 concrete and Fe415 steel. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a)i) | State the situations where you recommend doubly reinforced sections. | (3) |
| ii) | Describe how do you determine moment of resistance of doubly reinforce section. | (3) ………2/- |

-2-

| b) | Design a doubly reinforced section to carry the factored load of 45 KN/m over an effective span of 5 m. The beam is restricted to 250mm x 400mm. Take M20 concrete and Fe415 steel. |  |
| --- | --- | --- |
| c) | A beam 230mm x 450 mm (overall) consists of 3#12 on compression side and 3 # 16mm on tension side. Effective covers to both compression and tension reinforcement may be taken as 40mm each. If M25 concrete and Fe415 steel are used, determine the udl the beam can support at working condition over an effective span of 5 m. |  |
| Q.No.5. | Answer the following: |  |
| a) | Design the cantilever chajja over the window opening for the following data Projection of chajja from face of wall - 600mm LL on chajja - 0.75KN/m2 Load due to finishing - 0.50 KN/m2. M20 concrete and Fe415 steel are available. | (4) |
| b) | Design the slab for a hall measuring 4.5m x 5m. Two adjacent edges of slab are continuous and remaining two adjacent edges are discontinuous and slab is held down at corners. Take LL=2KN/m2 and Floor finishing =1KN/m2. Use M20 concrete and Fe1415 steel #### OR | (8) |
| b) | Design a dog legged stair to carry a LL of 2.5KN/m2 and finishing load of 1KN/m2. The stair is located in a hall measuring 2.2m x 4.8m. The floor to floor height is 3.0m. Use M25 concrete and Fe1415 steel. | (8) |
| Q.No.6. | Answer any one of the following: |  |
| a) | Design short circular column to carry an axial load of 750 KN adopting - Lateral ties - Helical reinforcement Use M25 concrete and Fe415 steel. | (5) (7) |
| b) | Design a square footing to carry an axial load of 500 KN from a column of 300mm x 300mm. The bearing capacity of soil is 150 KN/m2. Use M20 concrete and Fe415 steel. Carry out necessary checks for the section adopted. | (12) |

***********