## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(4297)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write short notes on methane layering. |  |
| b) | What are the standards of ventilation? |  |
| c) | How does laminar flow help in better ventilation? |  |
| d) | What are the salient features of homotropal ventilation? |  |
| e) | How reversal of air is achieved by centrifugal fan in underground mines? |  |
| f) | What is neutral line in boaster fan? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the physical chemical and physiological properties of carbonmonoxide. |  |
| b) | What are the sources of humidity in the underground coal mines? How does it effect the worker? |  |
| c) | Explain the various tests conducted by flame safety lamp to detect methane. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | - Differentiate between forcing & exhaust system of ventilation. - Write short notes on airlock. |  |
| b) | What is the purpose of splitting? Give advantages & disadvantages. |  |
| c) | Mean air temperature in a D.C. shaft 400m deep is 25ºC and in the U.C. shaft is 37ºC. Calculate i) the motive column and ii) the N.V.P assuming average barometric pressure in D.C. shaft to be 750mm of Hg. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the construction and working of a centrifugal fan. |  |
| b) | An airway 3m x 2.5m absorbs a ventilating pressure of 25mm w.g. Calculate by saving of water guage that can be affected by enlarging the roadways to 3.5m x 3m. Assume there is no change in quantity of air passing and coefficient of friction. Calculate the saving in ventilation cost per year if the quantity of air flowing is 4000 m3/minute. Energy cost per KWH = Rs. 2/-. |  |
| c) | Given below are the results of a pressure – quantity survey of main return airway: Pressure drop = 20mm w.g. Quantity of air flowing = 120 m3/min. Calculate (i) the resistance of the section of roadway in weissbach (ii) the total quantity of air in m3/min which would flow if an airway of equal resistance is placed in parallel to it. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | - Draw a neat sketch showing the ventilation of long heading by overlap method. Give important features. - Draw a neat sketch showing the pressure drop of a booster fan located in the inbye of intake gallery. |  |
| b) | i) What are the duties of ventilation officer? ii) In brief give the steps involved in pressure surveying? |  |
| c) | - What is the purpose of an evasee ? How is it designed? - Derive an equation for equivalent resistance of galleries connected in series. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the factors affecting Natural ventilation pressure? |  |
| b) | Differentiate between assentional & Discentional ventilation. |  |
| c) | Write short notes on flexible ducts. |  |
| d) | What are characteristics curves? Draw the characteristics curves of axial flow fan & explain it. |  |

***********