## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Refer to data provided at the end of the paper.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How do we identify pipe classes as A, B or C? |  |
| b) | What is the effect of temperature of the fluid conducted on the design of pipe? |  |
| c) | Give the symbols of following fittings with flanged connections (single line welded ends) - Blank flange at the end of Header - NRV (Non-return valve) - Top view of Tee-outlet up. |  |
| d) | What is “Glass Wool” & “Steel wool”? What are the characteristics of these materials? Where are they used? |  |
| e) | What is the normal pressure used in compressed air pipelines? Why? |  |
| f) | Two pipes of 2 m length each are connected by a long bend radius elbow to form an L-shape in the front view. Consider the pipe dia as 50 mm NB and intersection of centerlines as origin “O”. Draw the isometric view of this pipeline in single line representation. Also show dimensions in the view drawn. |  |
| Q.No.2. | Design a pipe material ASTM A333 GB to fabricate a pipeline to conduct a fluid at 320c; delivery pressure of 7 Kg/cm2 and discharge of 0.00682 m3/sec. The total length of this pipe is 2532 m and has in its length 1 no. of gate valve, 2 nos. of globe valves, 2 nos. of eccentric reducers and 12 nos. of short radius 90 elbow. | (12) |
| Q.No.3. | Answer the following: |  |
| a) | Draw the orthographic views and developed view of the portion of the pipeline whose isometric view is shown in Fig.1, Q.3.(a). | (8) |
| b) | Make a B.O.M. of the pipeline shown in fig.1,Q.3(a). Neglect all weights of pipe & pipe fittings. | (4) …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw a PFD for a general purpose compressed air plant having the following components arranged in sequence: Air compressor, Air cooler (Heat exchanger), Air dryer, Air filter and Receiver tank. Use appropriate symbols for drawing the circuit. |  |
| b) | Explain Thread and Hot Welding process of joining plastic pipes. |  |
| c) | Discuss the pre-heating and post weld heat treatment of ferrous pipes. |  |
| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Two pipe ends are to be fabricated to change the direction of flow to 60. If O.D. of the pipe is 50 mm. Draw the end profile and developed template to be used for cutting any one of the pipes. |  |
| b) | What is cladding of pipeline? Write the steps involved in insulation of pipelines. |  |
| c) | Write at least three characteristic points of each of the following plastic piping materials: - PVC - ABS. |  |
| Q.No.6. | Write short notes on any four of the following: | 4 x 3 = 12 |
| a) | Piping testing and certifying bodies in India |  |
| b) | Use of corrosion tables in design of pipes |  |
| c) | Symbols used in P & ID’s |  |
| d) | X-ray testing use in piping |  |
| e) | Material Inspection of pipes |  |
| f) | Any two types of pipe supports |  |
| g) | Types of Flanges used and shapes of gaskets cut for different flanges | Cont.. |