## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Process Control(4203)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the diagram of an elementary ON/OFF control system whose objective is to maintain the temperature of a room at some reference value Tref. |  |
| b) | Explain use of LAN’s in process control plants with an example. |  |
| c) | List three analog composite controller modes and state the expressions of each. |  |
| d) | Using a diagram explain the regulation of input (liquid) in a tank by a cascade control system. |  |
| e) | With a diagram, explain the use of a pneumatic actuator for converting pressure signal into mechanical shaft motion. |  |
| f) | Write a note on multiple loop controllers. |  |
| g) | What is differential gap? In which controller mode is it seen? |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the characteristics features of a discrete state process control system. Explain how sequence of events can be described of such system. |  |
| b) | What are relay sequences? Explain the operation of PLC. |  |
| c) | What is meant by process specification? Explain with example 2 basic categories of process hardware. |  |
| Q.No.3. | Answer the following: | 2 x 6 = 12 |
| a) | Explain the single and multiple speed floating controller mode. |  |
| b) | Write a note on: - Basic PLC counter - Process Equation with example. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the derivative controller mode using op-amps and explain the mode with characteristics and equation. |  |
| b) | State the general features of pneumatic controllers. Explain the implementation of integral controller mode using pneumatic techniques. |  |
| c) | Draw the circuit of two position controller using op-amps and show its neutral zone graph. Explain the use of alarms in digital circuits. |  |
| Q.No.5. | Answer the following: | 2 x 6 = 12 |
| a) | With a block diagram, explain the contents of a smart sensor. Explain how proportional mode can be implemented digitally using software. |  |
| b) | What is optimum control? Explain one method of process loop tuning. Give example and explain mechanical control element. | ………2/- |

-2-

| Q.No.6. | Answer any three of the following: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Explain the principle of fluid valve and lists the type of control valves. |  |
| b) | With example, explain electrical control elements. |  |
| c) | Explain direct and reverse acting pneumatic actuator with diagram. |  |
| d) | Write brief notes on: - Data logging - Pneumatic Amplifiers. |  |

***********