## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./EC/MED/E&EE

Subject: Electronic Circuits(4135)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Draw the hybrid equivalent circuit and write the V-I equations of a common base transistor. | (2) |
| b) | Define Sag or Tilt and write the expression for it in terms of lower cutoff frequency. | (2) |
| c) | Give any 2 points of comparison between class A, class B and class C operation. | (2) |
| d) | Define conversion efficiency of a power amplifier. | (1) |
| e) | Draw the circuit diagram of Emitter follower. | (1) |
| f) | Give the relation between gain of an amplifier with feedback and without feedback. | (1) |
| g) | Write the expression for voltage gain and input impedance of a Common Gate FET amplifier. | (2) |
| h) | Draw the circuit symbol of Triac and UJT. | (2) |
| i) | What are the advantages of an Electronic Switch? | (2) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Drive the expression for voltage gain, current gain and input resistance of Common Emitter transistor amplifier in terms of its hybrid parameters. |  |
| b) | Explain the response of an amplifier for a high frequency square wave input. |  |
| c) | With a neat circuit diagram explain the working of R-C coupled amplifier. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain various types of distortion in amplifiers. |  |
| b) | Draw the hybrid Pi model of CE transistor and explain the various resistors and hybrid Pi capacitances. |  |
| c) | What is f, f and fT of a transistor? Explain with neat diagram and relevant expressions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prove that the negative voltage series feedback brings in stability of gain and increases input impedance. |  |
| b) | With a neat circuit diagram explain the functioning of Hartley Oscillator. |  |
| c) | With a neat circuit diagram explain the working of RC phase shift oscillator. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | With a neat circuit diagram and waveforms explain the working of class B push pull power amplifier. |  |
| b) | List the advantages and disadvantages of a push pull circuit as compared to single ended power amplifier. |  |
| c) | Explain the working of a Common Source FET amplifier with a neat circuit diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat circuit diagram, working of Astable multivibrator. |  |
| b) | Explain the functioning of Schmitt Trigger circuit with a neat circuit diagram and waveforms. |  |
| c) | Briefly describe the construction and characteristics of SCR. |  |

***********