## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Industrial Instrumentation(4218)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Suggest a suitable type of orifice plate for measurement of flow of slurries. Give reason for your answer. |  |
| b) | Explain the Coriollis effect as applied to mass flow meter. |  |
| c) | List the various inherent error of Bourdon tube and suggest suitable remedies. |  |
| d) | Define Reynolds number and state the cut off limit for turbulent and laminar flow of viscous and non viscous liquids. |  |
| e) | Explain how lead compensation is carried out in temperature measurement using RTD. |  |
| f) | State an application where E/P converter is used. Explain the working of an E/P converter. |  |
| g) | Draw a diagram and explain the principle of a capacitance level indicator. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Explain the working of a turbine flow meter with help of a diagram. State its applications. |  |
| b) | State the advantages of positive displacement flow meters. Explain the working of any one type with the help of a diagram. |  |
| c) | State the advantages of electrical flow meter over differential pressure meter. Explain the working of ultrasonic flow meter with help of a diagram. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Sketch and explain the working of an optical pyrometer. State its applications. |  |
| b) | Explain the construction of RTD with help of diagram. Sketch the characteristics of a Pt 100 RTD. State the relationship between input and output of RTD. |  |
| c) | Give an example of a filled system thermometer. Draw a diagram and discuss the performance of such a system. |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Sketch and explain the working of a micro manometer and state its salient features. |  |
| b) | Stating the principle, sketch and explain the working of Pirani gauge. State its application. |  |
| c) | Explain the working of a DP cell. How does the DP transmitter differ from a DP cell? |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the working of any radiation method of level measurement. Discuss the performance. |  |
| b) | Discuss the need and techniques of servicing different types of level measuring instruments. |  |
| c) | Explain the working of a high pressure sight glass level measurement system. State the disadvantages. |  |
| Q.No.6. | Answer any two of the following: | 2 x 6 = 12 |
| a) | Draw the block diagram and explain the working of a practical closed loop temperature measurement system. |  |
| b) | Discuss the calibration techniques used in flow measurement. |  |
| c) | State application of V to I and I to V converter in Instrumentation. Draw circuits of each type. |  |

***********