## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Power Electronics & Drive Control(5102)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give three points of distinction between a DIAC & TRIAC. |  |
| b) | State any three important features of a Schottky diode. |  |
| c) | State the precaution to be taken during series operation of thyristor. How can it be achieved? |  |
| d) | State the Principles of operation of a.c. voltage controller and explain any one such principle of your choice with the help of neat circuit diagram & its input & output waveforms. |  |
| e) | State the different types of protections provided for thyristor. Also state the importance of snubber circuits. |  |
| f) | Classify static switches. Also state their advantages. |  |
| g) | Give two points of distinction between Buck & Boost switched mode regulator. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw Volt-Ampere characteristics of a thyristor and indicate on it the following: 1) VFBO 2) Latching current 3) Holding current 4) VRBR 5) OFF state 6) Conducting state. Also state the relationship between IL & IH. |  |
| b) | State the methods used for providing pulse gate triggering of thyristor. State its advantages. Give two points of distinction between UJT & PUT. | (4) (2) |
| c) | State the methods used for turning ON a thyristor. Which method is known as false method of triggering a thyristor and why? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of neat-circuit diagram and input as well as output waveforms, phase controlled converter operation. |  |
| b) | Draw a neat circuit diagram of single phase dual converter and draw its Input and output waveforms. Also explain its operation in brief. |  |
| c) | For a single phase full wave bridge type voltage controller operating resistive inductive load, draw - Its neat circuit diagram - Input voltage waveform - Output voltage and current waveform. Also explain its working in brief. |  |
| d) | How is it possible to generate 16 2/3 Hz frequency at the output for 50 Hz frequency at its input without changing voltage level? Explain the method used with the help of neat circuit diagram, input voltage waveform and output voltage waveform. |  |

………2/-

-2-

| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the significance of Free wheeling diode with the help of neat circuit diagram and waveforms. |  |
| b) | Classify the choppers. Explain the operation of any one of such chopper of your choice with the help of neat circuit diagram & operational diagram. |  |
| c) | Explain with the help of neat circuit diagram fundamental principle of operation of inverter. Also classify the inverters and state the basis of their classification. |  |
| d) | How is it possible to recover the trapped energy with a diode? Explain with the help of neat circuit diagram & waveforms |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify D.C. drives. Also explain in brief any one of such classified D.C. drive of your choice with the help of neat diagram circuit diagram & waveforms. |  |
| b) | State the methods used in speed control of D.C. drives. Explain any one of such method of your choice using appropriate thyristorised devices with the help of neat circuit diagram. |  |
| c) | State the possible modes of control of a d.c. chopper. Explain the principle of operation of any one such mode of control of your choice with the help of neat circuit diagram & waveforms. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is slip power? How is it possible to recover slip power by using appropriate method using thyrisorised devices? Explain with the help of neat circuit diagram. |  |
| b) | What is the significance of Voltage to Frequency control? Explain the methods of using such control with the help of thyristorised devices in speed control of a.c. drives. Draw appropriate circuit diagram. |  |
| c) | Explain with the help of neat circuit diagram and waveform the use of stator voltage control for speed control of single phase Induction motor. |  |

***********