## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Automobile Engg.

Subject: Thermal Engineering(4052)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Steam Tables and Mollier’s charts is permissible.

| Q.No.1. | Answer any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the function of blow-off cock. |  |
| b) | Explain the term “Enthalpy of steam”. |  |
| c) | Define indicated thermal efficiency and write its mathematical expression. |  |
| d) | Define thermal conductivity of a material. |  |
| e) | State the effect of air leakage into condenser. |  |
| f) | Define coefficient of performance of a refrigerator. |  |
| Q.No.2. | Answer any two of the following: | 2 x 6 = 12 |
| a) | What are the differentiating features between a water tube and a fire tube boiler? |  |
| b) | Determine the entropy per kg of superheated steam at a pressure of 20 bar and a temperature of 250C. Assume Cp for superheated steam as 2.2 KJ/Kg K. |  |
| c) | Describe briefly various types of artificial draught systems used in steam boilers. |  |
| Q.No.3. | Answer any two of the following: | 2 x 6 = 12 |
| a) | State the various methods of governing of steam turbine. Explain any one with a neat sketch. |  |
| b) | Explain with the help of sketch, the working of a four-stroke petrol engine. |  |
| c) | When is multi-stage compression used for air? What are its advantages? |  |
| Q.No.4. | Answer any two of the following: | 2 x 6 = 12 |
| a) | The walls of a room consists of parallel layers in contact of cement, brick and wood of thickness 20mm, 300mm and 10mm respectively. Find the quantity of heat that passes through the wall per m2 per minute, if the temperature of air in contact with the wall is 5C and 30C inside. The values of K for cement, brick and wood are 0.294,0.252 and 0.168 w/mk respectively. |  |
| b) | Calculate the vacuum efficiency; Given : Vacuum at steam inlet to condenser 700mm Hg Barometer reading 760mm Hg Hot well temperature 30C. |  |
| c) | Explain vapour compression refrigeration system with a neat sketch of P-h diagram. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | Explain the construction of any Superheater with a neat sketch. State the functions of Superheater. |  |
| b) | Using Mollier chart, find enthalpy drop and final condition of steam when it is expanded isentropically from an initial pressure of 30 bar and 360C to a pressure of 1 bar. |  |
| c) | An engine has piston diameter of 150mm, length of stroke 400mm and mean effective pressure 5.5 bar. The engine makes 120 explosions per minute. Determine the mechanical efficiency of the engine, if its brake power is 5KW. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Uses of compressed air |  |
| b) | Heat transfer by radiation |  |
| c) | Cooling tower |  |
| d) | Refrigerant – R-134A |  |
| e) | Losses occurring in a steam turbine |  |
| f) | Sensible heat load |  |

***********