## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engineering/EI/EC/MED/COMP/E&EE

Subject: Digital Electronics-II(4138)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Define resolution, accuracy and monotonicity for a DAC. |  |
| b) | What is the full form of PLD? What are the advantages of PLD’s over fixed function IC’s? |  |
| c) | What is multiplexer? Draw the block diagram of a digital multiplexer. |  |
| d) | Explain what is contact bounce problem in digital circuits. |  |
| e) | Draw the logic diagram of half adder and full subtractor and write the truth table. |  |
| f) | Show the 8 bit addition of +45 and –34 in 2’s complement representation. |  |
| g) | Add the following using BCD addition i) 73 and 29 ii) 113 and 61. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the diagram of a 4 bit binary adder-subtractor and explain its operation. |  |
| b) | With the help of neat logic diagram and truth table, explain the operation of full adder. |  |
| c) | Explain why multiplexer is called as an universal logic circuit. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the operation of decimal to BCD encoder with the help of logic diagram. |  |
| b) | Explain the working of BCD-to-seven segment decoder drivers. |  |
| c) | Explain why multiplexer is called as Universal logic circuit. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Draw the block diagram of counter type A to D converter and explain its operation. What are its drawbacks? |  |
| b) | Explain the operation of a 4 bit binary ladder type of DAC. What are its advantages over Weighted Resistor method? |  |
| c) | Explain the operation of successive approximation type ADC. What is the advantage of this method over counter method of ADC? |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the working of a TTL clock circuit using IC 7404. |  |
| b) | Explain with a neat circuit diagram the operation of monostable multivibrator using IC 555. |  |
| c) | Describe the following terms: - Mask programmable read only memory - Programmable read only memory - EPROM. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two: | 2 x 6 = 12 |
| a) | Explain the construction and operation of dynamic memory cell. |  |
| b) | Explain the construction and operation of OR matrix in PLA. |  |
| c) | Explain the architecture of a PAL using a neat diagram. |  |

………2/-

-2-

***********