## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Food Technology

Subject: Food Engineering-I(4265)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Explain Heat transfer coefficients and overall heat transfer coefficient with their significance in heat exchangers. |  |
| b) | Explain with neat sketch working of a tube heat exchanger. |  |
| c) | Discuss convection heat transfer with neat sketches and mathematical equations. |  |
| d) | What is DBT and WBT? How do they affect quality during storage of fruits? |  |
| e) | What will be yield when 2750 Kgs of skimmed milk with 9.75% solids is converted into powder with 1% moisture? |  |
| f) | What is flash evaporation? |  |
| g) | What is one tonne of refregeration? |  |
| h) | What is steam distillation? |  |
| i) | Define Relative humidity. |  |
| j) | What is the advantage of drying fruits in vacuum? |  |
| k) | What is the type of evaporator used for concentrating fruit juice? |  |
| l) | What is emissivity and absorptivity? |  |
| m) | Write three types of dryers commonly used in food industry. |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the process of heat transfer through a composite wall with neat sketch and mathematical equations. |  |
| b) | What are merits and demerits of tubular heat exchangers? |  |
| c) | Discuss with neat sketch advantage of counter current flow in a plate heat exchanger. |  |
| d) | Draw a neat sketch of a plate heat exchanger. Explain its use in pasteurization of milk. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain with a neat diagram working of a solar dryer. State advantages and disadvantages of a solar drier. |  |
| b) | Draw a typical refrigeration cycle and explain the process of refrigeration. |  |
| c) | Explain the principle of working of a freeze dryer. Discuss in brief merits and demerits of freeze drier. |  |
| d) | Explain working principle of Individual quick freezing with at least one example in food processing industry. |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Discuss drying curve with necessary sketches and mathematical expressions. |  |
| b) | Discuss the principle of vacuum evaporation. What advantage a vacuum drier offers in dehydration of foods? |  |
| c) | Explain the working of a Tray Drier. What are the merits and demerits of a tray drier? |  |
| d) | Discuss a typical continuous drier with working sketches and state one application in food processing. | …2/- |

-2-

| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| --- | --- | --- |
| a) | Explain working of an evaporator used for concentration of fruit juice. |  |
| b) | Explain the working of a spray drier with neat sketches. |  |
| c) | Explain the working of a Fluidised bed drier. State at least one application of Fluidised drier in Food Technology. |  |
| d) | How vacuum is generated in a vacuum evaporator. What is the quality advantage of a vacuum drying process? |  |
| Q.No.6. | Write short notes on any three of the following: | 3 x 4 = 12 |
| a) | Technology for extraction of spice oils |  |
| b) | Aroma recovery in multiple effect evaporator |  |
| c) | Psychometric charts |  |
| d) | Quick freezing of Foods |  |

***********