## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electronics Engg./ EC/MED/E&EE

Subject: Analog Electronics(4140)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain common mode rejection ratio in an op-amp and state its ideal and typical value. |  |
| b) | Sketch the output waveform of differentiator circuit for the following inputs: i)Sine wave ii)Square wave iii)Triangular wave. |  |
| c) | Draw the circuit diagram and frequency response of a first order wide band pass filter. |  |
| d) | Write a short note on IC 78xx series. |  |
| e) | Explain the term amplitude scaling in analog computers. |  |
| f) | Write a short note on Notch filter. |  |
| Q.No.2. | Answer any two: | 2 x 6 = 12 |
| a) | Explain briefly virtual ground concept in Operational amplifier and with the help of circuit diagram and necessary equations derive the expression for closed loop voltage gain of an inverting amplifier using virtual ground concept. |  |
| b) | With the help of a circuit diagram explain op-amp as a summing and averaging amplifier in non-inverting mode. |  |
| c) | Explain the following parameters of an op-amp: 1. Input offset voltage 2. Input bias current 3. Power supply rejection ratio 4. Slew rate. |  |
| Q.No.3. | Answer any two: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the operation of Wein bridge oscillator using op-amp. |  |
| b) | Explain the operation of triangular wave generator. |  |
| c) | Draw the circuit diagram of an Integrator using op-amp and derive the expression for its output voltage. Also list the disadvantages of the basic Integrator circuit. |  |
| Q.No.4. | Answer any two: | 2 x 6 = 12 |
| a) | Design a 2nd order high pass filter with a cut off frequency of 1KHz. Also draw the frequency response for the same. |  |
| b) | Explain the Operation of an Schmitt trigger circuit. |  |
| c) | Explain time scaling in analog computer with the help of an example. |  |
| Q.No.5. | Answer any two: | 2 x 6 = 12 |
| a) | Explain PLL with the help of a block diagram. |  |
| b) | Explain PLL as AM demodulator. |  |
| c) | Write a note on short circuit protection in voltage regulators. |  |
| Q.No.6. | Answer any three: | 3 x 4 = 12 |
| a) | In what way is the voltage follower a special case of the non-inverting amplifier? |  |
| b) | Write a short note on Quadrature Oscillator using op-amp. |  |
| c) | Explain the working of a positive clipper with positive reference voltage. |  |
| d) | Write a note on the functional block diagram of IC 723. |  |

***********