## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: A.C. Machines(4113)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the features of salient pole and non-salient pole field structure. |  |
| b) | What do you mean by hunting of synchronous machine? |  |
| c) | How does change in supply voltage and frequency affect the performance of induction motor? |  |
| d) | List down the conditions to be satisfied for proper synchronisation of alternator. Also give advantages of parallel operation of alternator. |  |
| e) | Explain the working principle of synchronous motor. |  |
| f) | 1 induction motor is not self starting. Explain why. Also list the various methods to make it starting. |  |
| g) | Define starting torque, running torque, pull in torque and pull out torque. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of 3 slip ring induction motor. |  |
| b) | Give the factor on which rotor torque depends. Give the equation for running torque and starting torque. Explain when the torque will be maximum. |  |
| c) | A 3, 4 pole, 50 Hz Induction motor has rotor resistance of 0.04 per phase. The maximum torque occurs at speed of 1200 rpm. Calculate the starting torque as a percentage of maximum torque. |  |
| d) | Define slip and derive for frequency of rotor current. Also calculate the rotor speed and rotor frequency for 3 induction motor with 2 poles when connected to 400 V, 50Hz supply at slip of 4%. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how to achieve high starting torque in slip ring and Squirrel cage induction motor. |  |
| b) | Compare induction motor and synchronous motor. |  |
| c) | Explain with a neat sketch starting of induction motor with star-delta starter. Also give the need of starter for starting 3 induction motor. |  |
| d) | Explain following methods of speed control of motors: - Frequency variation method - Voltage variation method. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define voltage regulation of alternator. List the different method of determining voltage regulation and explain any one method in detail. |  |
| b) | Give the classification of armature winding of alternator and explain. |  |

………2/-

-2-

| c) | Explain the following: - Synchronous generator on load with unity power factor load. - Synchronous generator on load with lagging power factor load. - Synchronous generator on load with leading power factor load. |  |
| --- | --- | --- |
| d) | A 100 KVA, 3000volts, 50Hz, 3 star connected alternator having armature resistance of 0.2, field current of 40A produces short circuit current of 200A and open circuit emf of 1040 volts (line value). Calculate the full load voltage regulation at 0.8 power factor lagging & 0.8 power factor leading. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the effect of variation of excitation, change of prime mover input on parallel connection of alternator. |  |
| b) | What is the effect of armature reaction on alternator at unity power factor, lagging power factor and leading power factor? |  |
| c) | Give advantages, disadvantages and application of synchronous motor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of universal motor. How to provide the speed control in such motor and give its application? |  |
| b) | Explain the construction and working of reluctance motor. |  |
| c) | Explain the effect of change in supply voltage on torque and slip. |  |
| d) | Explain the methods of starting of 1 Induction motor by split phase method and shaded pole method. |  |

***********