## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Question No.1 is compulsory. Answer any five from the remaining

questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Shallow water effect on resistance |  |
| b) | Frictional resistance |  |
| c) | Geometric similarity |  |
| d) | Propeller materials |  |
| e) | Water-cooled stern tube |  |
| f) | Kort Nozzle |  |
| Q.No.2. | A ship is 125m long, 16m beam and floats at a draught of 7.8m. Its block co-efficient is 0.72. Calculate the power required to overcome frictional resistance at 17.5 knots if n=1.825 and f=0.423. Use Taylor’s formula for wetted surface with c=2.55. | (12) |
| Q.No.3. | Answer the following: |  |
| a) | Draw a typical screw propeller drawing and explain the various views. | (8) |
| b) | A propeller of 4.5m pitch turns at 120 rev/min and drives the ship at 15.5 knots. If the wake fraction is 0.30, calculate the apparent slip and real slip. | (4) |
| Q.No.4. | Answer the following: |  |
| a) | A propeller of diameter 5.8m provides a speed of 16 knots to ship at 120 rpm. Efficiency of the propeller is 68%. If froude wake fraction is 0.22 and thrust deduction fraction is 0.15, find PE, PT, actual thrust and torque for a delivered power of 1860 KW. | (8) |
| b) | Differentiate between water-cooled and oil-cooled stern tube. | (4) |
| Q.No.5. | Answer the following: |  |
| a) | What is the need for boring stern tube in place after welding it to the hull? | (6) |
| b) | Explain the terms w.r.t towing tank: i)Corresponding speed ii)Froude’s law of comparison. | (6) |
| Q.No.6. | A model 5.2m in length has a total resistance of 67 newtons when towed at 3.5 knots. The wetted surface of the model is 4.25 m2. Determine the PE for the ship of similar form and length 131m at the corresponding speed. fmodel = 1.728 sw fship = 1.418 sw | (12) |

……2/-

-2-

| Q.No.7. | Answer the following: |  |
| --- | --- | --- |
| a) | Explain in brief the concept of controllable pitch propeller. | (6) |
|  | ### OR |  |
| a) | Explain the vertical axis propeller. | (6) |
| b) | When does a ship need to be provided a reduction and reversing gear box? Explain the functioning of reversing and reduction gear box. | (6) |
|  | ### OR |  |
| b) | Explain why ocean going ship are not provided with gear box. | (6) |

***********