## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2012 Examinations

Programme: Electrical Engg./E&EE

Subject: Transformers(4106)

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is the use of a tap changer in transformers? Give the advantages of OLTC over off load Tap changer. |  |
| b) | What is meant by all day efficiency of a transformer? |  |
| c) | What are the effects of change in voltage on the performance of transformer? |  |
| d) | Explain the construction and purpose of breather in a transformer. |  |
| e) | List down the conditions necessary to be satisfied for operation of 3 transformer in parallel. |  |
| f) | What is the reason for using laminated core? State the material used for the core. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Derive an expression for the induced emf in a transformer. |  |
| b) | Explain leakage flux and its effect on the working of transformer. |  |
| c) | With a neat sketch, give the differences between distributed cross over and spiral winding. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain phasor diagram of transformer when it is operating under no load. Modify the above phasor diagram to take into account loading effect on the transformer. |  |
| b) | Draw the equivalent circuit of a transformer referred to secondary side. State the practical usefulness of such equivalent circuit. |  |
| c) | A 30 KVA 2400/120V, 50HZ transformer has a high voltage winding resistance of 0.1 and a leakage resistance of 0.22 . The low voltage winding resistance is 0.035 and the leakage reactance is 0.012. find the equivalent winding resistance, reactance and impedance referred to the i) high voltage side and ii) the low voltage side. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a 25 KVA, 2000/200V, single phase transformer the iron and full load copper losses are 350 and 400W respectively. Calculate the efficiency at unity power factor on i) full load ii) half full load. |  |
| b) | Define voltage regulation of a transformer. Give the expressions for voltage regulation and also give the significance of voltage regulation. |  |
| c) | Explain vector groups of transformers. |  |

………2/-

-2-

| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | What are the special features of the welding transformers? |  |
| b) | Explain scott connection of two transformers. |  |
| c) | Two 1 phase transformers A and B rated at 250 KVA each are operated in parallel on both sides. Percentage impedance for A and B are (1+j6) and (1.2+j4.8) respectively. Compute the load shared by each when the total load is 500KVA at 0.8 p.f. lagging. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out the different types of losses in a transformer and briefly explain each of them. |  |
| b) | A 300-KVA single phase transformer is designed to have a resistance of 1.5 and maximum efficiency occurs at a load of 173.2 KVA. Find its efficiency when supplying full load at 0.8 p.f. lagging at normal voltage and frequency. |  |
| c) | Give the detailed construction of a single phase autotransformer and its uses. |  |
| d) | Two 100KW, single phase transformers are connected in parallel both on the primary and secondary. One transformer has an ohmic drop of 0.5% at full load and an inductive drop of 8% at full load current. The other has an ohmic drop of 0.75% and inductive drop of 2%. Show how will they share a load of 180 KW at 0.9 power factor. |  |

***********